# Supplementary Information

Bayesian Neural Surrogates Unify Uncertainty Propagation, Sensitivity Attribution and Calibration for Coupled Multi-Physics Reactor Analysis

Yinuo Chen, Sicheng Wang, Xiaojing Liu*, Tengfei Zhang*

# Contents

I.   Supplementary Figures  ..........................................  (below) II.  Supplementary Tables  .........................................  (below) III. Supplementary Notes A. BNN training details  ......................................  (below) B. Sobol sensitivity methodology  .............................  (below) C. MCMC posterior calibration details  ........................  (below) D. Forward UQ extensions  .....................................  (below) E. Coupled simulation and material model details  .............  (below) References  ........................................................  (below)

# I. Supplementary Figures

## Fig. S1 | BNN surrogate workflow.

[Source: figures/fig1_workflow.png]

Schematic of the end-to-end probabilistic analysis framework. The BNN surrogate is trained on coupled OpenMC-FEniCS data and produces a posterior predictive distribution. This single distribution serves as the shared probabilistic layer for forward uncertainty propagation (Section 2.2), Sobol sensitivity attribution (Section 2.3), and MCMC posterior calibration (Section 2.4), without retraining between analyses.

## Fig. S2 | Parity plots for all 15 outputs.

[Source: figures/bank/B1_stress_parity.png, figures/bank/B2_keff_parity.png, figures/bank/B3_thermal_parity.png]

(a-c) Predicted versus true values for the 435 test samples, grouped by output category. (a) Coupled peak global stress (R2 = 0.935). (b) Coupled keff (R2 = 0.633). (c) Coupled thermal outputs (max fuel temperature, max monolith temperature). Shaded regions show the 90% predictive interval. Dashed line: y = x identity.

[TO GENERATE: expand to all 15 outputs in multi-panel figure]

## Fig. S3 | Probability integral transform (PIT) histograms.

[Source: results/accuracy/pit_bnn-baseline.png, results/accuracy/pit_bnn-data-mono-ineq.png]

PIT histograms for the reference surrogate (a) and the physics- regularized BNN (b) across the five primary outputs. A uniform PIT distribution indicates well-calibrated predictive intervals. The physics-regularized BNN shows slightly improved calibration relative to the reference.

## Fig. S4 | Training curves.

[TO GENERATE from training logs]

(a) ELBO loss versus epoch for the physics-regularized BNN. (b) KL divergence component. (c) Negative log-likelihood component. (d) Monotonicity loss component. The KL annealing schedule transitions from 0 to full weight over the first 100 epochs.

## Fig. S5 | Out-of-distribution (OOD) coverage comparison.

[Source: results/ood/ood_coverage_comparison.png]

90% predictive interval coverage for in-distribution versus four feature-tail subsets. Stress retains coverage above 0.75 across all tails. keff coverage degrades substantially on the αbase tail (R2 drops to approximately 0.29), indicating that the Sobol and posterior conclusions for keff are valid only within the training range of this dominant parameter.

## Fig. S6 | Epistemic uncertainty ratio under distribution shift.

[Source: results/ood/ood_epistemic_ratio.png]

Ratio of epistemic to total predictive variance for in-distribution versus OOD tail subsets. Elevated epistemic ratios on the αbase tail confirm that the BNN correctly signals increased model uncertainty outside the training domain.

## Fig. S7 | Sobol index convergence with sample size.

[Source: results/sensitivity/sobol_convergence_bnn-phy-mono_stress.png, results/sensitivity/sobol_convergence_bnn-phy-mono_keff.png]

(a,b) First-order Sobol index S1 as a function of sample size NS (256 to 4096) for stress (a) and keff (b), with 90% CI bands across 50 replications. Indices stabilise by NS = 2048, confirming convergence at the chosen NS = 4096.

## Fig. S8 | Coupled versus decoupled output distributions.

[Source: figures/bank/C1_stress_coupling.png, figures/bank/C2_keff_distribution.png]

(a) Histogram comparison of coupled and decoupled stress distributions from 20,000 LHS samples. Coupling shifts the mean downward by approximately 40 MPa and narrows the standard deviation by approximately 28%. (b) keff distribution: the decoupled keff is effectively constant; all variability originates from thermal-expansion feedback on core geometry.

## Fig. S9 | MCMC trace plots (representative cases).

[Source: results/posterior/trace_bnn-phy-mono_case00.png, results/posterior/trace_bnn-phy-mono_case06.png, results/posterior/trace_bnn-phy-mono_case12.png]

Trace plots for three representative benchmark cases spanning low (case 0), near-threshold (case 6), and high stress (case 12). Each panel shows the four calibrated parameters across 4 independent chains (8000 iterations, first 2000 discarded as burn-in). Visual mixing and stationarity support convergence.

## Fig. S10 | Prior versus posterior marginal distributions.

[Source: figures/bank/E1_prior_posterior.png]

Prior (uniform, grey) versus posterior (coloured) marginal distributions for the four calibrated parameters, shown for six representative benchmark cases. Eintercept and αbase show the strongest posterior contraction, consistent with their dominant Sobol indices for stress and keff respectively.

## Fig. S11 | Posterior predictive distributions.

[Source: figures/bank/E3_posterior_predictive.png]

BNN-predicted stress distributions at the prior (blue) versus posterior (orange) parameter sets for representative benchmark cases. The posterior-predictive distribution concentrates around the observed true value (dashed line), demonstrating successful calibration.

## Fig. S12 | Posterior 90%-CI coverage summary.

[Source: figures/bank/E2_posterior_coverage.png]

Per-case and per-parameter 90%-CI coverage across all 18 benchmark cases. Overall coverage: 66/72 = 0.917. Cases with missed coverage are predominantly in the αslope parameter, which has the weakest Sobol sensitivity and is therefore least constrained by the observations.

## Fig. S13 | Epistemic versus aleatoric variance decomposition.

[Source: results/uncertainty_decomposition/epi_vs_ale_scatter_bnn-phy-mono.png]

Scatter plot of epistemic (weight-uncertainty) versus aleatoric (observation-noise) variance for each of the 435 test samples. Stress is dominated by aleatoric variance (the model is confident in its mean prediction), while keff shows elevated epistemic variance reflecting the more limited training signal.

# II. Supplementary Tables

## Table S1 | Material parameters: definitions, nominal values, and priors.

| Parameter | Symbol | Nominal value | Unit | Prior range (+-10%) |
| --- | --- | --- | --- | --- |
| Young's mod. | Eintercept | 2 x 1011 | Pa | [1.8, 2.2] x 1011 |
| intercept |  |  |  |  |
| Young's mod. | Eslope | -7 x 107 | Pa/K | [-7.7, -6.3] x 107 |
| slope |  |  |  |  |
| Poisson's | nu | 0.31 | - | [0.279, 0.341] |
| ratio |  |  |  |  |
| CTE base | αbase | 1 x 10-5 | 1/K | [0.9, 1.1] x 10-5 |
| CTE slope | αslope | 5 x 10-9 | 1/K^2 | [4.5, 5.5] x 10-9 |
| Ref. cond. | kref | 23.2 | W/(m*K) | [20.88, 25.52] |
| Ref. temp. | Tref | 923.15 | K | [830.84, 1015.47] |
| Cond. slope | kslope | 1/75 | W/(m*K^2) | [0.012, 0.0147] |

All priors are independent uniform: theta_k ~ U[0.9*theta_nom, 1.1*theta_nom]. E(T) = Eslope * T + Eintercept. alpha(T) = αbase + αslope * T. k(T) = kref + kslope * (T - Tref).

## Table S2 | BNN hyperparameter search space and optimal values.

| Hyperparameter | Search range | Optimal (bnn-phy-mono) |
| --- | --- | --- |
| Width | [64, 256] | 254 |
| Depth | [2, 8] | 2 |
| Learning rate | [1e-4, 1e-2] | 9.81 x 10-4 |
| Weight decay | [1e-9, 1e-5] | 8.09 x 10-8 |
| Batch size | {16, 32, 64} | 32 |
| Epochs | [200, 500] | 399 |
| Gradient clip | [0.1, 2.0] | 0.674 |
| w_data | [0.1, 5.0] | 1.855 |
| Prior sigma | [0.1, 2.0] | 0.478 |
| KL weight | [1e-5, 1e-2] | 1.93 x 10-4 |
| w_mono | [0.01, 1.0] | 0.212 |
| rho_abs_min | [0.01, 0.5] | 0.123 |
| mono_topk | [10, 100] | 53 |

Search: Optuna with 200 trials; objective: validation RMSE. Activation: SiLU throughout. Output heads: mean + log-variance (heteroscedastic).

## Table S3 | BNN ablation study: four model variants on five primary outputs.

Metric: R2 on coupled peak global stress (mean +/- s.d., 5 seeds).

| Model variant | Stress R2 | Stress PICP | keff R2 |
| --- | --- | --- | --- |
| Reference (ELBO only) | 0.936 +/- 0.007 | 0.995 +/- 0.002 | 0.633 +/- 0.260 |
| Data-monotone | 0.932 +/- 0.005 | 0.998 +/- 0.002 | 0.635 +/- 0.260 |
| Physics-regularized (primary) | 0.935 +/- 0.007 | 0.987 +/- 0.007 | 0.633 +/- 0.258 |
| Data+inequality | 0.924 +/- 0.007 | 1.000 +/- 0.001 | 0.617 +/- 0.249 |

The physics-regularized BNN achieves comparable R2 to the reference while producing tighter (lower PICP) but still well-calibrated intervals. The data+inequality variant shows the most conservative intervals (PICP = 1.0) at slightly reduced R2.

## Table S4 | Complete Sobol indices (S_1 and S_T) for stress and k_eff.

Physics-regularized BNN, NS = 4096, 50 replications.

| Parameter | Stress S1 [90% CI] | Stress ST [90% CI] |
| --- | --- | --- |
| Eslope | 0.037 [-0.046, 0.100] | 0.039 [0.038, 0.039] |
| Eintercept | 0.548 [0.498, 0.572] | 0.566 [0.562, 0.571] |
| nu | 0.063 [0.034, 0.097] | 0.068 [0.067, 0.069] |
| αbase | 0.210 [0.171, 0.254] | 0.214 [0.212, 0.217] |
| αslope | 0.038 [-0.013, 0.091] | 0.047 [0.046, 0.049] |
| Tref | 0.019 [-0.025, 0.055] | 0.032 [0.031, 0.033] |
| kref | 0.065 [0.014, 0.118] | 0.073 [0.072, 0.075] |
| SS316_alpha | -0.004 [-0.068, 0.034] | 0.005 [0.005, 0.006] |

| Parameter | keff S1 [90% CI] | keff ST [90% CI] |
| --- | --- | --- |
| Eslope | 0.010 [-0.052, 0.062] | 0.005 [0.004, 0.005] |
| Eintercept | 0.008 [-0.065, 0.071] | 0.006 [0.006, 0.007] |
| nu | 0.029 [-0.035, 0.086] | 0.018 [0.018, 0.019] |
| αbase | 0.771 [0.747, 0.788] | 0.767 [0.758, 0.776] |
| αslope | 0.195 [0.112, 0.250] | 0.198 [0.196, 0.201] |
| Tref | 0.010 [-0.023, 0.054] | 0.011 [0.011, 0.012] |
| kref | 0.000 [-0.063, 0.048] | 0.006 [0.006, 0.007] |
| SS316_alpha | 0.003 [-0.059, 0.074] | 0.009 [0.009, 0.009] |

[NOTE: S_1 values from sobol_convergence.csv (N_S=4096, 50 reps, bootstrap CI); S_T values from sobol_results.csv (same run, Jansen estimator). S_T CIs are narrower because they use direct replicate quantiles rather than bootstrap.]

## Table S5 | Posterior calibration: 18 benchmark case summary.

| Case | Category | True stress | Accept rate | 90%-CI coverage | Post-mean |
| --- | --- | --- | --- | --- | --- |
| (MPa) | (4 params) | stress err. |  |  |  |
| 0 | low | 110.6 | 0.619 | 3/4 | 8.3 |
| 1 | low | 118.7 | 0.612 | 4/4 | 24.6 |
| 2 | low | 110.7 | 0.620 | 2/4 | 9.0 |
| 3 | low | 103.7 | 0.618 | 4/4 | 8.8 |
| 4 | low | 114.5 | 0.621 | 4/4 | 3.1 |
| 5 | low | 102.0 | 0.616 | 3/4 | 23.4 |
| 6 | near | 127.6 | 0.609 | 4/4 | 20.3 |
| 7 | near | 127.9 | 0.586 | 4/4 | 26.6 |
| 8 | near | 127.0 | 0.611 | 4/4 | 5.9 |
| 9 | near | 123.5 | 0.613 | 4/4 | 3.5 |
| 10 | near | 128.9 | 0.609 | 3/4 | 4.5 |
| 11 | near | 129.3 | 0.610 | 4/4 | 3.8 |
| 12 | high | 169.5 | 0.592 | 4/4 | 0.2 |
| 13 | high | 176.0 | 0.591 | 4/4 | 12.1 |
| 14 | high | 193.4 | 0.598 | 3/4 | 13.7 |
| 15 | high | 163.4 | 0.591 | 4/4 | 3.6 |
| 16 | high | 142.8 | 0.582 | 4/4 | 10.5 |
| 17 | high | 197.6 | 0.583 | 4/4 | 5.4 |

Overall: acceptance 0.58-0.62 (mean 0.60); 90%-CI coverage 66/72 = 0.917; posterior-mean stress MAE = 10.4 MPa over 102-198 MPa range.

## Table S6 | External baseline comparison.

See Table S10 for the full external baseline comparison (MC-Dropout and Deep Ensemble scoring rules across all five primary outputs).

## Table S7 | Computational cost breakdown.

[Source: results/speed/budget_matched_risk.csv]

| Operation | Time | Platform |
| --- | --- | --- |
| Coupled HF solve (1 case) | 2266 s | EPYC 9654 / 96 cores |
| BNN single draw (M=50) | 1.29e-2 s | GPU |
| BNN batched (batch=1024, per sample) | 1.34e-5 s | GPU |
| 20,000-sample forward UQ | ~0.3 s | GPU |
| Sobol (50 reps x 40,960 evals) | ~minutes | GPU |
| 18-case MCMC (8000 iter x 4 chains) | ~minutes | GPU |
| Speedup (single draw vs HF) | 1.76e5 |  |
| Speedup (batched vs HF) | 1.69e8 |  |

## Table S8 | Physics consistency: monotonicity violation rates on primary outputs (435 test samples, discrete +/-delta perturbation test).

| Physics pair (input -> output, sign) | Ref. BNN | Data-mono | Phy-reg | Data+ineq |
| --- | --- | --- | --- | --- |
| Eintercept -> Max stress (+) | 0% | 0% | 0% | 0% |
| Eslope -> Max stress (+) | 0% | 0% | 0% | 0% |
| αbase -> Max stress (+) | 0% | 0% | 0% | 0% |
| αbase -> keff (-) | 0% | 0% | 0% | 0% |
| αslope -> keff (-) | 0% | 0% | 0% | 0% |
| SS316_k_ref -> Max fuel temp (-) | 0% | 0% | 0% | 0% |
| SS316_k_ref -> Max monolith temp (-) | 0% | 0% | 0% | 0% |
| SS316_k_ref -> Max stress (-) | 0% | 0% | 0% | 1.1% |
| SS316_alpha -> Max fuel temp (-) | 36.3% | 39.5% | 35.6% | 53.6% |

All high-confidence pairs show 0% violations across all models. The SS316_alpha -> fuel temperature pair (medium confidence) shows 36-54% violations, reflecting a weaker relationship that the training data does not consistently support. The SS316_k_ref -> stress violation for the data+inequality BNN (1.1%) has maximum magnitude 1.89 MPa, negligible relative to the stress range (102-198 MPa).

Inequality constraints (max >= average temperature, stress >= 0): 0% violations for all four BNN variants.

## Table S9 | Epistemic vs aleatoric uncertainty decomposition on test set.

| Output | Ref. BNN | Phy-reg BNN |
| --- | --- | --- |
| Epi% | Ale% | Epi% |
| keff | 12.0 | 88.0 |
| Max fuel temp | 12.0 | 88.0 |
| Max monolith T | 11.2 | 88.8 |
| Max stress | 27.7 | 72.3 |
| Wall expansion | 34.1 | 65.9 |

Aleatoric uncertainty dominates across all outputs (58-90%). Physics regularization slightly reduces epistemic uncertainty for temperature outputs while marginally increasing it for stress and wall expansion.

## Table S10 | External baseline: full comparison of scoring rules.

| Model | Output | RMSE | R2 | CRPS | NLL | ECE |
| --- | --- | --- | --- | --- | --- | --- |
| MC-Dropout | keff | 0.0003 | 0.856 | 0.0002 | -6.502 | 0.152 |
| Fuel temp | 4.497 | 0.590 | 2.624 | 3.352 | 0.183 |  |
| Monolith T | 5.061 | 0.590 | 2.951 | 3.472 | 0.182 |  |
| Max stress | 7.659 | 0.934 | 4.519 | 3.592 | 0.118 |  |
| Wall exp. | 0.0008 | 0.999 | 0.002 | -3.831 | 0.197 |  |
| Deep Ensemble | keff | 0.0003 | 0.828 | 0.0002 | -6.476 | 0.142 |
| Fuel temp | 4.495 | 0.590 | 2.634 | 3.326 | 0.190 |  |
| Monolith T | 5.061 | 0.590 | 2.962 | 3.439 | 0.185 |  |
| Max stress | 7.644 | 0.934 | 4.502 | 3.586 | 0.112 |  |
| Wall exp. | 0.0006 | 0.999 | 0.002 | -3.841 | 0.197 |  |

The BNN surrogates achieve comparable point-prediction accuracy (RMSE, R2) to the external baselines but substantially better probabilistic calibration (ECE) on temperature outputs. On stress all methods perform similarly.

## Table S11 | Prior sensitivity: posterior properties under six prior configurations (physics-regularized BNN, case 06, near-threshold).

| Prior variant | Coverage (4 params) | Mean KL vs canonical | Accept rate |
| --- | --- | --- | --- |
| Canonical | 100% | 0.00 (reference) | 0.61 |
| Diffuse (2x) | 100% | 0.15-0.22 | 0.63 |
| Tight (0.5x) | 50-83% | 0.64-1.28 | 0.57 |
| Flat (uniform) | 100% | 0.24-0.38 | 0.64 |
| Shift (+0.5s) | 67-100% | 0.06-0.36 | 0.60 |
| Shift (-0.5s) | 83-100% | 0.08-0.28 | 0.61 |

The canonical and diffuse priors both achieve full coverage. The tight prior over-constrains the posterior, reducing coverage to 50-83% with KL divergences up to 1.28, indicating that the canonical prior width is near-minimal adequate. The flat prior maintains coverage but produces wider posteriors.

## Table S12 | Noise sensitivity: posterior properties as observation noise varies (physics-regularized BNN, case 06).

| Noise fraction | Eintercept post. std | Accept rate | Coverage |
| --- | --- | --- | --- |
| 0.5% | 1.26e10 | 0.58 | 100% |
| 1.0% | 1.28e10 | 0.59 | 100% |
| 2.0% (default) | 1.32e10 | 0.61 | 100% |
| 5.0% | 1.36e10 | 0.63 | 100% |
| 10.0% | 1.40e10 | 0.65 | 83% |

Posterior widths increase monotonically with noise level. Coverage remains at 100% until 10% noise, where αslope drops to 83%. Acceptance rate increases appropriately as the likelihood broadens.

## Table S13 | Data efficiency: surrogate test metrics as a function of training set size (2 seeds).

| Train fraction | N_train | RMSE (mean+/-std) | R2 (mean+/-std) |
| --- | --- | --- | --- |
| 25% | 507 | 4.93 +/- 0.04 | 0.768 +/- 0.002 |
| 50% | 1015 | 4.66 +/- 0.11 | 0.782 +/- 0.005 |
| 75% | 1522 | 4.68 +/- 0.01 | 0.784 +/- 0.002 |
| 100% | 2029 | 4.62 +/- 0.08 | 0.783 +/- 0.005 |

The BNN achieves 96% of peak RMSE performance with only 25% of the training data. Marginal improvement from 50% to 100% data is < 1%.

## Fig. S14 | Prior sensitivity sweep.

[Source: posterior/bnn-phy-mono/prior_sensitivity/

prior_sensitivity_bnn-phy-mono.png]

(a-d) Posterior marginal distributions for the four calibrated parameters under six prior configurations: canonical, diffuse (sigma -> 2*sigma), tight (sigma -> 0.5*sigma), flat (uniform), shift_pos (mu + 0.5*sigma), shift_neg (mu - 0.5*sigma). The tight prior produces visibly narrower posteriors that miss the true parameter values in several cases (Table S11).

## Fig. S15 | Observation noise sensitivity.

[Source: posterior/bnn-phy-mono/noise_sensitivity/

noise_sensitivity_bnn-phy-mono.png]

Posterior width and acceptance rate as a function of assumed observation noise fraction (0.5%-10%). Posterior standard deviations increase monotonically; acceptance rates increase appropriately as the likelihood broadens (Table S12).

## Fig. S16 | Data efficiency curve.

[Source: results/data_efficiency/data_efficiency_curve.png]

Test RMSE, NLL and R2 as a function of training set size for the reference and physics-regularized BNN (2 seeds per point). Near- saturated performance is achieved at 25% data utilisation (Table S13).

## Fig. S17 | Epistemic vs aleatoric uncertainty decomposition.

[Source: results/uncertainty_decomposition/

uncertainty_decomposition_bar.png, epi_vs_ale_scatter_bnn-phy-mono.png]

(a) Stacked bar chart showing the epistemic (blue) and aleatoric (orange) fractions of total predictive variance for each primary output, comparing the reference and physics-regularized BNN. (b) Per-sample scatter of epistemic versus aleatoric standard deviation for the physics-regularized BNN; most samples lie below the diagonal, confirming aleatoric dominance.

# III. Supplementary Notes

# Supplementary Note A: BNN training details

### A.1. Evidence lower bound (ELBO) derivation

The BNN posterior over weights w is approximated by a mean-field variational distribution q(w) = product_i N(w_i | mu_i, sigma_i^2), where each weight w_i has its own variational mean mu_i and variance sigma_i^2. The variational parameters are optimised by minimising the negative evidence lower bound:

where p(w) = N(0, sigma_p^2 I) is the isotropic Gaussian prior and p(D | w) is the data likelihood. The KL divergence between two Gaussians admits a closed form; the data-fit term is approximated by a single Monte Carlo sample per mini-batch (Bayes-by-Backprop [10]).

The reparameterisation trick enables gradient computation: w = mu + sigma * epsilon, where epsilon ~ N(0, I). The variance parameter sigma is parameterised as sigma = log(1 + exp(rho)) to ensure positivity, with rho_abs_min clamping rho >= rho_abs_min to prevent posterior collapse.

### A.2. Heteroscedastic observation noise

The network outputs both a mean mu(theta) and a log-variance log sigma^2_obs(theta) per output channel. The per-output Gaussian log-likelihood is:

log p(y_j | theta, w) = -0.5 * [log sigma^2_{obs,j} +

(y_j - mu_j)^2 / sigma^2_{obs,j}]

This heteroscedastic formulation allows the BNN to model input- dependent observation noise, separating it from epistemic (weight) uncertainty.

### A.3. KL annealing schedule

The KL weight beta is annealed from 0 to its target value kl_weight = 1.93 x 10-4 over the first 100 epochs using a linear schedule:

beta(t) = min(t / 100, 1.0) * kl_weight

This prevents the KL term from dominating early training before the data-fit term has found a reasonable region of weight space.

### A.4. Physics-prior monotonicity constraints

The physics-regularized BNN adds an auxiliary loss term that penalises violations of known monotonicity relationships derived from the constitutive laws. For each specified (input, output) pair where the physics dictates a positive (or negative) partial derivative, the loss is:

L_mono = (1/K) sum_{k=1}^{K} max(0, -sign * d_mu/d_theta_k)

where the gradient is computed by automatic differentiation and K is the number of sampled training points per batch (controlled by mono_topk). The monotonicity loss is weighted by w_mono = 0.212 in the total loss.

The sign constraints are derived from the constitutive physics: - Stress increases with Eintercept (positive sign), because

sigma = C(E, nu) * epsilon and C is linear in E.

- Stress increases with αbase (positive sign), because thermal

strain epsilon_th = alpha * DeltaT serves as the loading source.

These are physics-prior constraints, not inequality priors. The inequality-constraint variant (tested in ablation, Supplementary Table S3) is a separate formulation.

### A.5. Gradient clipping and initialisation

Gradient norms are clipped at 0.674 per parameter group. Weight means mu are initialised with Xavier uniform; rho values are initialised uniformly in [-5, -4], corresponding to small initial variance. These choices stabilise early training when the KL term is annealed in.

# Supplementary Note B: Sobol sensitivity methodology

### B.1. Sobol variance decomposition

For a square-integrable function f(theta_1, ..., theta_d) with independent inputs, the total variance V = Var[f] can be decomposed as:

V = sum_i V_i + sum_{i<j} V_{ij} + ... + V_{1,...,d}

where Vi = Var[E[f | theta_i]] is the first-order variance due to theta_i alone, and higher-order terms capture interactions. The first-order Sobol index is Si = Vi / V, and the total-order index is ST,i = 1 - V_{~i} / V, where V_{~i} = Var[E[f | theta_{~i}]] is the variance explained by all inputs except theta_i.

### B.2. Saltelli cross-matrix scheme

Indices are estimated using the Saltelli sampling scheme. Two independent N x d matrices A and B are drawn from the joint prior. For each input i, a cross-matrix A_B^{(i)} is constructed by replacing the i-th column of A with the i-th column of B. The model is evaluated at all N(d+2) points. First-order and total-order indices are then estimated from the resulting output vectors.

### B.3. Jansen estimator

The first-order index is estimated using the Jansen estimator:

S_1,i = 1 - (1/2N) sum_{j=1}^{N} [f(A_B^{(i)})_j - f(B)_j]^2 / V

The total-order index uses:

S_{T,i} = (1/2N) sum_{j=1}^{N} [f(A)_j - f(A_B^{(i)})_j]^2 / V

The Jansen estimator is preferred for its lower variance compared to the Sobol estimator, particularly for indices near zero.

### B.4. Replication-based confidence intervals

The entire Sobol analysis (sample generation, model evaluation, index estimation) is independently repeated R = 50 times. The 90% confidence interval for each index is constructed from the 5th and 95th percentiles of the R replications. A factor is declared a stable contributor only if the lower bound of its 90% CI exceeds zero.

The BNN posterior predictive mean (averaged over M = 30 weight samples) serves as the deterministic function f(theta) in the Sobol framework. Using the predictive mean rather than a single weight sample ensures that the sensitivity analysis captures parametric sensitivity rather than BNN sampling noise.

# Supplementary Note C: MCMC posterior calibration details

### C.1. Likelihood formulation

The likelihood for a single benchmark case with observed outputs y_obs = (y_1, ..., y_J) is:

sigma_{obs,j}^2 + sigma_{BNN,j}^2(theta))

where mu_j(theta) is the BNN posterior predictive mean (averaged over M = 20 weight samples), sigma_{BNN,j}^2 is the BNN aleatoric variance, and sigma_{obs,j}^2 is the observation noise variance (2% of the true value). The J = 5 primary outputs are: coupled peak global stress, coupled peak fuel temperature, coupled peak monolith temperature, coupled wall expansion, and keff.

### C.2. Proposal distribution

A symmetric Gaussian random-walk proposal is used:

theta* = theta_current + epsilon, epsilon ~ N(0, Sigma_proposal)

where Sigma_proposal = diag(scale_1^2, ..., scale_4^2). The proposal scales are set to produce acceptance rates in the range 0.2-0.7 for each parameter. The scales were calibrated by short pilot runs of 1000 iterations.

### C.3. Chain configuration

For each benchmark case: - 4 independent chains with dispersed initialisations drawn from the prior - N_total = 8000 iterations per chain - Burn-in: first 2000 iterations discarded - Thinning: every 5th post-burn-in sample retained - Effective posterior: 1200 samples per chain, 4800 total per case

The four calibrated parameters are Eintercept, αbase, αslope, and kref, selected as the four with the largest Sobol total-order indices across stress and keff. The remaining four parameters (Eslope, nu, Tref, kslope) are fixed at their true values.

### C.4. Convergence diagnostics

Convergence is assessed by: 1. Visual inspection of trace plots (Supplementary Fig. S9) 2. Rank-normalised split-R-hat (Vehtari et al. 2021): R-hat_max = 1.017 across all 72 parameter-case combinations, well below the 1.1 threshold 3. Geyer initial-positive-sequence effective sample size: ESS_min = 352 (all cases) 4. Rank histograms (Supplementary Fig. S9): uniform distribution confirms adequate between-chain mixing 5. Acceptance rates: range 0.58-0.62, mean 0.605 (Table S5)

Acceptance rates in the 0.44-0.66 optimal range for four-dimensional targets (Roberts & Rosenthal 2001) indicate adequate mixing.

### C.5. Benchmark case selection

The 18 benchmark cases are drawn from the 435-sample test split to cover a representative range of coupled stress levels: - 6 low-stress cases (true stress 102-119 MPa) - 6 near-threshold cases (true stress 123-129 MPa) - 6 high-stress cases (true stress 143-198 MPa)

The threshold for categorisation is the primary safety threshold of 131 MPa. For each case, the "observation" is the true HF output vector perturbed by 2% multiplicative Gaussian noise, simulating realistic measurement uncertainty.

# Supplementary Note D: Forward UQ extensions

### D.1. Posterior predictive mean versus full predictive draw

The main-text forward UQ (Section 2.2) propagates uncertainty through the BNN posterior predictive mean y = E_w[mu_w(theta)], which isolates parametric uncertainty from the BNN's own epistemic and aleatoric spread. As a consistency check, we also propagate through full predictive draws (sampling both weight uncertainty and observation noise):

y_full ~ (1/M) sum_{m=1}^{M} N(mu_{w^(m)}(theta),

sigma^2_{obs,w^(m)}(theta))

The full-draw distribution is wider because it includes BNN uncertainty. However, the means and percentile rankings are consistent between the two protocols, confirming that the parametric sensitivity structure is robust to the choice of predictive protocol.

Table S14 compares the two protocols for the physics-regularized surrogate at the reference 1-sigma input perturbation (sigma_k = 1.0).

Table S14. Forward UQ protocol comparison (bnn-phy-mono, sigma_k = 1.0, N = 20,000 LHS samples, 50 MC weight draws per sample)

## Statistic                  Posterior mean     Full predictive draw protocol           protocol

Mean stress (MPa)          163.55             163.55 Std stress (MPa)           —                  33.62 5th percentile (MPa)       —                  113.15 50th percentile (MPa)      —                  161.11 95th percentile (MPa)      —                  222.74 Mean pred. sigma (MPa)     —                  13.00 - epistemic component    —                   7.33 - aleatoric component    —                  10.69 P(stress > 131 MPa)        —                   0.840

The posterior-mean protocol isolates parametric uncertainty: the mean stress prediction is identical between protocols (163.55 MPa), confirming that the BNN's epistemic spread does not bias the central tendency. The full-draw protocol is wider (std = 33.62 MPa vs. 13.0 MPa for per-sample predicted sigma) because it adds weight-space and observation-noise variance to the parametric spread. For the reference surrogate, the same comparison yields a stress mean of 164.21 MPa and full-draw std of 33.75 MPa, consistent within 0.4%.

The agreement in means between protocols confirms that Sobol indices computed from the posterior-mean protocol (Section 2.3) are not artefacts of the predictive protocol choice.

### D.2. Latin hypercube sampling

Latin hypercube sampling (LHS) is used rather than simple random sampling because LHS ensures more uniform coverage of the input space, reducing sampling variance for a given sample size. With N = 20,000 samples and d = 8 inputs, the LHS design partitions each input dimension into 20,000 equal-probability intervals and ensures exactly one sample per interval per dimension.

To verify that N = 20,000 provides adequate coverage, we examine the stability of Sobol first-order indices as the LHS base sample size N_base increases from 256 to 8,192 (each with 20 bootstrap repeats). For the dominant input Eintercept on coupled stress, the physics-regularized surrogate yields:

N_base    S1_mean   S1_std    95% CI ------    -------   ------    ------ 256      0.547     0.044    [0.497, 0.603] 512      0.546     0.028    [0.509, 0.577] 1024      0.559     0.020    [0.525, 0.586] 2048      0.553     0.025    [0.513, 0.580] 4096      0.548     0.023    [0.498, 0.572] 8192      0.543     0.011    [0.527, 0.560]

The S1 central estimate stabilises at N_base >= 2,048 (range of means: 0.543-0.553), while the bootstrap standard deviation decreases monotonically from 0.044 to 0.011, confirming that the N = 20,000 LHS sample used for main-text forward UQ is well within the convergent regime. The same convergence pattern holds for αbase on keff (S1 stable at 0.769-0.778, std decreasing from 0.019 to 0.012).

This convergence behaviour is consistent across both the reference and physics-regularized surrogates (see Fig. S10 for convergence curves), confirming that the LHS design provides reliable sensitivity estimates at the chosen sample size.

# Supplementary Note E: Coupled simulation and material model details

### E.1. Material constitutive models

The SS316 monolith is the structural component that simultaneously bears thermal stress and transmits heat between fuel and heat pipes. All eight uncertain parameters enter the calculation through the constitutive models for this single material:

Structural mechanics parameters. The temperature-dependent Young's modulus is modelled as E(T) = Eslope * T + Eintercept. Eintercept sets the baseline stiffness (nominal 2 x 1011 Pa). Eslope captures temperature softening (nominal -7 x 107 Pa/K). Poisson's ratio nu (nominal 0.31) characterises the multi-axial stress state. These three parameters control stress through the generalised Hooke's law, sigma = C(E, nu) * epsilon, where C is the fourth-order stiffness tensor.

Thermal deformation parameters. The coefficient of thermal expansion is modelled as alpha(T) = αbase + αslope * T. αbase is the expansion coefficient at the reference condition (nominal 1 x 10-5 K^{-1}). αslope captures its temperature dependence (nominal 5 x 10-9 K^{-2}). These two parameters serve a dual role: they govern thermal strain epsilon_th = alpha * DeltaT (the loading source for thermal stress) and they determine the magnitude of core geometric expansion (the primary pathway through which material properties affect the neutronic balance).

SS316 thermal transport parameters. The thermal conductivity is modelled as k(T) = kref + kslope * (T - Tref). The reference conductivity kref has nominal value 23.2 W/(m*K). The reference temperature Tref has nominal value 923.15 K. The temperature-dependence coefficient kslope has nominal value 1/75 W/(m*K^2). Together, these control the temperature gradient across the monolith. For consistency with the released code and data files, the source names the three quantities SS316_k_ref, SS316_T_ref and SS316_alpha respectively.

Fuel thermal conductivity, heat-pipe working-fluid properties and other non-monolith parameters are held at their nominal values because they do not participate in the stress constitutive law and their influence on keff is mediated primarily through the monolith temperature field, which the eight selected parameters already control.

### E.2. Picard iteration and convergence

The OpenMC-FEniCS coupling proceeds as follows: 1. Initialise with the eight material parameters and reference geometry. 2. OpenMC computes keff and the spatially resolved fission power distribution. 3. FEniCS solves for the temperature field, thermal expansion displacement and stress field. 4. The updated geometry and temperature-dependent material cross- sections are fed back to OpenMC. 5. Steps 2-4 repeat until convergence.

Convergence criterion: maximum temperature difference between consecutive iterations < 1 K. An under-relaxation factor of 0.1 is applied to stabilise the iteration [9].

### E.3. Output definitions

Each HF evaluation produces 15 scalar outputs:

Decoupled (7 outputs, before coupling convergence): 1. Average fuel temperature 2. Peak fuel temperature 3. Peak monolith temperature 4. Peak global stress 5. Monolith mean temperature 6. Core height after expansion 7. Wall expansion

Coupled (8 outputs, after full convergence): 8.  keff 9.  Average fuel temperature 10. Peak fuel temperature 11. Peak monolith temperature 12. Peak global stress 13. Monolith mean temperature 14. Core height after expansion 15. Wall expansion

The posterior likelihood uses 5 primary outputs: items 8, 10, 11, 12, 15.

### E.4. Dataset generation

n = 2900 samples were generated by Latin hypercube sampling over the 8-dimensional prior. The split (seed = 2026): - Training: 2029 samples (~70%) - Validation: 436 samples (~15%) - Test: 435 samples (~15%)

The fixed split is reused across all experiments for reproducibility.

# Supplementary References

[10] Blundell C, et al. Weight uncertainty in neural networks. ICML 2015; pp. 1613-1622. [9]  Zhang T, Chen Y, Wang S, Liu X. Design-oriented multiphysics analysis and surrogate modelling of a heat-pipe-cooled reactor. Energy. 2025. (in press)