Probabilistic Neural Surrogates for Uncertainty-to-Risk Analysis in Coupled Multi-Physics Systems: Application to a Heat-Pipe-Cooled Reactor

概率神经代理模型在耦合多物理场系统不确定性–风险分析中的应用——以热管冷却反应堆为例

Draft v3  |  Generated 2026-04-16 13:24

# 作者 / Authors

Yinuo Chen¹ᵃ˒ᵇ, Sicheng Wang²ᵇ˒ᶜ, Xiaojing Liu³ᵃ˒ᵇ and Tengfei Zhang⁴ᵃ˒ᵇ˒*

a  School of Nuclear Science and Engineering, Shanghai Jiao Tong University, Shanghai, 200240, China b  Shanghai Digital Nuclear Reactor Technology Integration Innovation Center, Shanghai, 200240, China c  College of Smart Energy, Shanghai Jiao Tong University, Shanghai, 200240, China

*  Corresponding author: zhangtengfei@sjtu.edu.cn

# 摘要 / Abstract

We develop a Bayesian neural network (BNN) probabilistic surrogate for uncertainty-to-risk analysis in safety-critical coupled multiphysics systems and demonstrate it on a heat-pipe-cooled reactor (HPR) with fully coupled neutronics–thermal–structural feedback. The BNN is trained on coupled OpenMC–FEniCS simulations and maps uncertain material parameters to a posterior predictive distribution over the coupled outputs, natively combining epistemic (weight) uncertainty with aleatoric (observation) uncertainty. On a held-out test set the stress coefficient of determination reaches R² = 0.910 and the 90% interval coverage reaches PICP₉₀ = 0.998. Forward Monte Carlo propagation shows that multiphysics coupling reshapes the stress uncertainty distribution and reduces its standard deviation by approximately 28% relative to the decoupled prediction. Sobol analysis identifies largely independent dominant pathways for stress (Eintercept, S₁ ≈ 0.56) and keff (αbase, S₁ ≈ 0.74). MCMC calibration against noisy observations yields a posterior 90% interval coverage of 88.9% for the four calibrated parameters and stratifies exceedance risk at an adopted 131 MPa design-screening threshold. The surrogate runs orders of magnitude faster than the coupled solver and thereby enables end-to-end probabilistic risk analysis that is otherwise infeasible at the high-fidelity level.

本文面向安全敏感的耦合多物理系统中的不确定性–风险分析，构建了一套贝叶斯神经网络（Bayesian Neural Network, BNN）概率代理建模框架，并以具备完整中子–热–力耦合反馈的热管冷却反应堆（HPR）作为工程验证对象。BNN 在耦合 OpenMC–FEniCS 仿真数据上训练，将材料不确定输入映射为耦合输出的后验预测分布，天然统一认识论（权重）不确定性与偶然（观测）不确定性。在留出测试集上，应力决定系数 R² = 0.910，90% 区间覆盖率 PICP₉₀ = 0.998。前向蒙特卡洛传播表明，多物理耦合重塑了应力的不确定性分布，使其标准差相对解耦预测降低约 28%。Sobol 分析识别出应力与 keff 的主导作用路径基本独立（应力：Eintercept S₁ ≈ 0.56；keff：αbase S₁ ≈ 0.74）。MCMC 在含噪观测下的后验标定对 4 个被标定参数给出 90% 区间覆盖率 88.9%，并在所采用的 131 MPa 设计筛选阈值下分层评估超阈风险。代理模型较耦合求解器快多个数量级，从而使高保真层面不可行的端到端概率风险分析得以实现。

# 1. 引言 / Introduction

[第一段：对象与计算问题]

Heat-pipe-cooled reactors (HPRs) are compact, monolithic-core systems in which neutronics, heat transfer and structural deformation are coupled through two-way feedback rather than arranged in a one-way sequence [1]. Temperature fields drive thermal expansion and stress, and the resulting geometric changes feed back into neutron leakage and power distribution. Under such closed feedback, uncertainty in material properties cannot be propagated independently within any single physics, and coupled uncertainty transmission, rather than single-field prediction, becomes the central computational problem.

热管冷却反应堆（HPR）是将中子学、传热与结构变形通过双向反馈紧密耦合的紧凑型基体式堆芯系统，而非单向串联过程 [1]。温度场驱动基体热膨胀与应力演化，由此产生的几何变化又反馈至中子泄漏与功率分布。在这种闭合反馈下，材料属性不确定性无法在任一物理域内独立传播，耦合条件下的不确定性传递——而非单场点预测——由此成为核心计算问题。

[第二段：国际研究现状与统一概率计算层的空白]

High-fidelity multiphysics analysis of compact heat-pipe microreactors has become an active international direction. MOOSE-based toolchains that couple neutronics, thermo-mechanics and heat-pipe physics have been demonstrated on both KRUSTY-class and larger heat-pipe reactor designs [2,3]. Parallel studies based on commercial thermal–structural solvers repeatedly identify stress concentration and structural integrity as recurring safety concerns in monolithic-core configurations [4,5]. Beyond the HPR literature, nuclear computational science is now developing stochastic analysis, surrogate generation and probabilistic workflow capabilities around large-scale simulators, both within the MOOSE ecosystem [6,7] and through emerging digital-twin-oriented pipelines [8]. These advances nonetheless stop short of a unified probabilistic computational layer that, within a single coupled framework, simultaneously supports fast surrogate prediction, forward uncertainty propagation, global sensitivity attribution and observation-conditioned posterior updating.

紧凑型热管微堆的高保真多物理分析已成为国际上的活跃方向。基于 MOOSE 的工具链已在 KRUSTY 类与更大功率热管堆设计上实现了中子学、热–力学与热管物理的耦合 [2,3]。基于商用热–力求解器的平行研究反复指出，应力集中与结构完整性是基体式堆芯构型中的持续性安全关注点 [4,5]。除 HPR 专门研究外，核工程计算领域也正在围绕大规模仿真器发展随机分析、代理建模与概率工作流能力，其中既包括 MOOSE 生态中的相关模块 [6,7]，也包括面向数字孪生的初步管线 [8]。然而这些进展仍未形成一个统一的概率计算层，尚不能在同一耦合框架内同时支撑快速代理预测、前向不确定性传播、全局敏感性归因与观测条件下的后验更新。

[第三段：空白的意义——高层表述]

This gap matters because uncertainty in HPRs propagates across coupled neutronic, thermal and structural channels rather than within a single physics. The same material parameter can influence multiple outputs simultaneously, and coupled feedback may either amplify or attenuate the resulting response variability. Risk-oriented analysis therefore requires a coupled probabilistic treatment rather than decoupled or single-physics evaluation.

这一空白之所以重要，是因为 HPR 中的不确定性并非局限于单一物理，而是在耦合的中子、热工与结构通道间同时传播。同一材料参数可同时作用于多个输出量，而耦合反馈既可能放大、也可能衰减最终响应变异。因此，风险导向分析必须采用耦合概率处理，而不能依赖解耦或单场评估。

[第四段：工程风险锚点与 131 MPa 筛选阈值]

Structural stress is the practically relevant risk variable in this class of systems. Design analyses of compact heat-pipe-cooled cores have reported nominal peak stresses on the order of 169 MPa [9], well within the range in which material-parameter uncertainty can shift the exceedance probability against a design-screening threshold. We therefore adopt 131 MPa as such a screening threshold for the present analysis and frame the central question as how uncertainty in material properties translates into exceedance risk at that threshold, rather than as point prediction at nominal conditions.

在该类系统中，结构应力是具工程意义的风险变量。紧凑型热管冷却堆芯的设计分析已报道约 169 MPa 的标称峰值应力 [9]，处于材料参数不确定性足以改变超阈概率的量级。本文据此取 131 MPa 作为设计筛选阈值，并将核心问题表述为"材料参数不确定性如何转化为该阈值下的超阈风险"，而不是标称工况下的点预测。

[第五段：高保真直接评估与现有代理方法的边界]

Direct evaluation based on repeated high-fidelity simulation is computationally prohibitive: each coupled OpenMC–FEniCS solve requires on the order of 10³ s per sample, so forward Monte Carlo over an 8-dimensional material-parameter space, and Bayesian calibration requiring comparable likelihood counts, are both infeasible at the high-fidelity level. Standard surrogates close only part of this gap. Gaussian-process and polynomial-chaos methods become increasingly constrained in strongly nonlinear, moderately high-dimensional coupled settings; physics-informed neural networks are difficult to formulate when the governing structure is expressed through iterative solver exchange rather than a single closed residual; and purely deterministic data-driven networks scale but, without calibrated predictive intervals, cannot carry downstream risk analysis on their own. Bayesian neural networks (BNNs) address the calibration gap by placing a distribution over the network weights, so that the posterior predictive distribution naturally carries both the epistemic uncertainty arising from limited training data and the aleatoric uncertainty arising from the observation process.

直接基于高保真重复求解的评估在计算上不可行：每次 OpenMC–FEniCS 耦合求解约需 10³ 秒，因而在 8 维材料参数空间上进行前向蒙特卡洛传播、以及需要相近量级似然评估的贝叶斯标定，均无法在高保真层面完成。常规代理方法仅能部分弥合这一差距：高斯过程与多项式混沌方法在强非线性、中高维耦合设置下受维度与光滑性假设限制；物理信息神经网络难以为以迭代求解器交换而非单一闭合残差表达的控制结构直接构造损失；纯确定性数据驱动神经网络虽具扩展性，但在缺乏校准预测区间时无法独立承担下游风险分析。贝叶斯神经网络（BNN）通过对网络权重引入分布，使后验预测分布天然同时承载来自有限训练数据的认识论不确定性与来自观测过程的偶然不确定性，从而弥补校准层面的缺口。

[第六段：本文所做——BNN 概率代理与端到端不确定性–风险管线]

In this work we develop a Bayesian neural network probabilistic surrogate for uncertainty-to-risk analysis in a coupled neutronics–thermal–structural system and demonstrate it on a heat-pipe-cooled reactor. The BNN is trained on coupled OpenMC–FEniCS data and predicts, for multiple physics outputs, posterior predictive means together with input-dependent predictive uncertainties that combine weight-space and observation-noise contributions. Around this surrogate we assemble an end-to-end uncertainty-to-risk pipeline that combines forward Monte Carlo propagation, Sobol sensitivity with bootstrap confidence intervals, and MCMC posterior calibration. The main contributions are threefold. First, we introduce a BNN probabilistic surrogate that jointly predicts posterior predictive means and calibrated predictive intervals for the coupled outputs, with an explicit decomposition into epistemic and aleatoric components. Second, we integrate the BNN into a unified probabilistic computational layer that consistently treats the same predictive distribution across forward UQ, sensitivity attribution and posterior calibration. Third, we demonstrate that this computational layer connects expensive coupled multiphysics simulation to tractable, risk-oriented analysis in safety-critical coupled systems.

本文面向耦合中子–热–力系统，构建了一种贝叶斯神经网络概率代理模型，并以热管冷却反应堆作为工程验证对象。BNN 在耦合 OpenMC–FEniCS 数据上训练，对多个物理输出同时给出后验预测均值与输入依赖的预测不确定性，其预测方差可显式分解为权重空间的认识论分量与观测噪声的偶然分量。围绕该代理，本文组织端到端的不确定性–风险分析管线，联合前向蒙特卡洛传播、带 Bootstrap 置信区间的 Sobol 敏感性分析与 MCMC 后验标定。主要贡献有三：其一，提出 BNN 概率代理模型，对耦合输出同时给出均值与经校准的预测区间，并显式区分认识论与偶然不确定性；其二，将 BNN 嵌入统一的概率计算层，使前向 UQ、敏感性归因与后验标定共用同一预测分布；其三，论证该计算层将昂贵的耦合多物理仿真衔接至安全敏感耦合系统中可行的风险导向分析。

# 2. 结果 / Results

## 2.1 代理精度 / Surrogate Accuracy

The BNN surrogate places a distribution over network weights and yields a posterior predictive distribution for each output; architecture, prior, and inference scheme are detailed in Methods. The surrogate was evaluated on the held-out test set (n = 435). For the primary safety metric, coupled steady-state maximum global stress, the BNN achieved R² = 0.910, RMSE = 8.93 MPa, and MAE = 6.98 MPa. The 90% posterior-predictive interval coverage PICP₉₀ = 0.998 and mean interval width MPIW₉₀ = 64.3 MPa confirm that the predictive uncertainty is well calibrated (Table 2, Fig. 2A). For keff, the BNN achieved R² = 0.825, PICP₉₀ = 1.000, and MPIW₉₀ = 2.90 × 10⁻³ (≈ 290 pcm), indicating similarly reliable interval coverage (Table 2).

本文采用的 BNN 代理模型在网络权重上引入分布，对每个输出量给出后验预测分布，具体架构、先验与推断方案见方法节。在留出测试集（n = 435）上，耦合稳态最大全局应力的预测精度为 R² = 0.910，RMSE = 8.93 MPa，MAE = 6.98 MPa； 90% 后验预测区间覆盖率 PICP₉₀ = 0.998，平均区间宽度 MPIW₉₀ = 64.3 MPa，表明预测不确定性具有良好的校准特性（表 2，图 2A）。keff 的预测精度为 R² = 0.825，PICP₉₀ = 1.000，MPIW₉₀ = 2.90 × 10⁻³（约 290 pcm），区间覆盖同样可靠（表 2）。

Table 2 compares the constraint-regularized BNN against two reference points: (i) an ELBO-only BNN baseline without the physics constraints (internal label `bnn-baseline`), which isolates the contribution of the monotonicity and inequality terms; and (ii) the deterministic constraint-regularized heteroscedastic MLP of the authors' prior work [Zhang et al. 2025, Energy; internal label `data-mono-ineq`], which quantifies the incremental cost or benefit of introducing Bayesian weight uncertainty. Stress R² across the three models lies in a narrow band [0.910, 0.929]; the constraint-regularized BNN attains PICP₉₀ = 0.998 for stress — conservative over-coverage relative to the nominal 90% target — while providing, unlike the deterministic surrogate, an explicit decomposition of predictive variance into epistemic and aleatoric components, which is required by the downstream risk analysis (Sections 2.2–2.4).

表 2 将约束正则化 BNN 与两个对照对象并列：(i) 去掉物理约束的纯 ELBO BNN 基线（内部标号 `bnn-baseline`），用于分离单调性+不等式项的贡献； (ii) 作者前期工作中的确定性约束正则化异方差 MLP（Zhang et al. 2025, Energy；内部标号 `data-mono-ineq`），用于量化引入贝叶斯权重不确定性的增量成本或收益。三个模型的应力 R² 位于 [0.910, 0.929] 的窄带内；约束正则化 BNN 在应力上取得 PICP₉₀ = 0.998，相对 90% 的名义目标呈现保守超覆盖；同时——与确定性代理不同——给出预测方差的认识论/偶然显式分解，为下游风险分析（2.2–2.4 节）所必需。

The BNN was therefore adopted as the primary surrogate for all subsequent analyses, on the basis of calibration quality and the availability of a posterior predictive distribution that is internally consistent across forward UQ, sensitivity attribution and posterior calibration. Supplementary observations under out-of-distribution conditions are reported in the Appendix as supporting evidence and do not by themselves drive the model-selection decision.

综合上述结果，BNN 代理被选为后续全部分析的主模型；该选择基于校准质量以及 BNN 后验预测分布在前向 UQ、敏感性归因与后验标定间的内在一致性。附录另报告了分布外条件下的补充观测，作为辅助支撑证据，并不单独决定模型选择。

[TABLE 2: 测试集精度三列对比——(i) HeteroMLP data-mono-ineq (0411 canonical) (ii) bnn-baseline (iii) bnn-data-mono-ineq；主要输出量的 R²/RMSE/MAE/PICP₉₀/MPIW₉₀，增加一列 "epistemic 分量 (是/否)"]

[FIG 2: (A) 应力预测-真值散点图（含 90% 后验预测区间，BNN 主模型） (B) 三模型 stress RMSE + MPIW 并列条形对比]

## 2.2 前向不确定性传播 / Forward Uncertainty Propagation

Forward propagation of 20,000 Latin hypercube samples through the BNN posterior predictive mean function (y = E_w[μ_w(θ)], isolating parametric uncertainty from the BNN's epistemic+aleatoric predictive spread; see Methods) reveals how multi-physics coupling reshapes the output uncertainty landscape (Table 4, Fig. 3).

对 20,000 个拉丁超立方样本通过 BNN 后验预测均值函数（y = E_w[μ_w(θ)]，隔离参数不确定性与 BNN 自身认识论+偶然预测展宽；详见方法节）的前向传播，揭示了多物理耦合如何重塑输出不确定性格局（表 4，图 3）。

For coupled steady-state stress, the prior-predictive distribution has a mean of 162.5 MPa and standard deviation of 35.5 MPa (5th, 50th, 95th percentiles: 107.4, 160.5, 224.1 MPa). The corresponding decoupled (single-pass) prediction has a mean of approximately 202.4 MPa and standard deviation of approximately 49.2 MPa (derived from the test-set iter1/iter2 variance ratio var(σ_coupled)/var(σ_decoupled) = 0.52 and the observed mean shift Δ = −39.9 MPa between passes). The coupling feedback therefore reduces the stress standard deviation by ~28% (from ~49.2 to 35.5 MPa), suggesting that multi-physics coupling attenuates a substantial fraction of the material-parameter-driven stress variability that a decoupled analysis would predict.

耦合稳态应力的先验预测分布均值为 162.5 MPa，标准差 35.5 MPa（5、50、 95 分位数分别为 107.4、160.5、224.1 MPa）。对应的解耦（单轮）预测均值约为 202.4 MPa，标准差约为 49.2 MPa（由测试集 iter1/iter2 方差比 var(σ_coupled)/var(σ_decoupled) = 0.52 与观测到的平均差 Δ = −39.9 MPa 推得）。耦合反馈使应力标准差降低约 28%（从约 49.2 降至 35.5 MPa），表明多物理耦合衰减了解耦分析所预测的材料参数驱动应力变异中相当一部分。

For coupled keff, the standard deviation is 1.07 × 10⁻³ (approximately 107 pcm), with a mean of 1.1036. Under the current LHS design, the first-iteration (decoupled) keff is effectively constant across all samples (standard deviation ~5 × 10⁻¹⁴), because the 8 sampled material parameters do not enter the neutron cross-section tables at the reference geometry. The coupled keff variability is therefore attributable to thermo-mechanical feedback: thermal expansion modifies core geometry, which is expected to affect neutron leakage and produce the observed ~107-pcm spread. No decoupled-to-coupled compression ratio for keff is quoted, as the decoupled value carries no meaningful parametric uncertainty.

耦合 keff 标准差为 1.07 × 10⁻³（约 107 pcm），均值 1.1036。在当前 LHS 设计下，第一轮（解耦）keff 在所有样本中实质上为常数（标准差约 5 × 10⁻¹⁴），因为 8 个采样的材料参数在参考几何条件下不进入中子截面表。因此耦合 keff 的变异可归因于热力反馈：热膨胀改变堆芯几何形状，预期将影响中子泄漏，产生观测到的约 107 pcm 展宽。由于解耦值不携带有意义的参数不确定性，不引用 keff 的解耦–耦合压缩比。

The exceedance probabilities P(σ > τ) at a range of design-screening thresholds provide a direct risk characterisation:

不同设计筛选阈值下的超阈概率 P(σ > τ) 提供直接的风险表征：

| τ (MPa) | P(σ > τ) |
| --- | --- |
| 110 | 0.941 |
| 120 | 0.893 |
| 131 | 0.816 |

At the 131 MPa design-screening threshold, 81.6% of prior samples exceed the threshold — indicating that under the ±10% prior uncertainty in material parameters, exceedance is the expected rather than exceptional outcome. The forward-UQ predictive distribution has 5th, 50th, and 95th percentiles of 107.4, 160.5, and 224.1 MPa respectively, so the nominal peak stress of 169 MPa reported by Zhang et al. (2025) lies between the median and p95 of the prior-predictive stress distribution. Threshold levels above 131 MPa (including 169 and 200 MPa) were not evaluated in the current threshold sweep and are deferred to the appendix.

在 131 MPa 设计筛选阈值处，81.6% 的先验样本超过阈值——确认在材料参数 ±10% 先验不确定性下，超阈为预期结果。前向 UQ 预测分布的 5、50、 95 分位数分别为 107.4、160.5、224.1 MPa，故 Zhang et al.（2025）报道的标称峰值应力 169 MPa 落在该先验预测分布的中位数与 p95 之间。当前阈值扫描不包含 131 MPa 以上的水平（包括 169、200 MPa），相关结果后续在附录补充。

[TABLE 4: 前向 UQ 统计——应力和 keff 的均值/标准差/超阈概率]

[FIG 3: (A) 应力直方图：解耦 vs 耦合 (B) 耦合 keff 分布 + 阈值线]

## 2.3 全局敏感性分析 / Global Sensitivity Analysis

Sobol variance-based sensitivity analysis (Saltelli sampling, Jansen estimator, N = 4096, 50 independent replications; see Methods) identified distinct dominant pathways for the two primary outputs (Table 5, Fig. 4).

基于方差的 Sobol 敏感性分析（Saltelli 采样、Jansen 估计量、N = 4096、 50 次独立重复；详见方法节）识别出两个主要输出的不同主导路径（表 5，图 4）。

For coupled steady-state stress, Eintercept is the dominant first-order factor (S₁ = 0.558, 90% CI: [0.551, 0.565]), accounting for approximately 59% of the first-order variance. αbase ranks second (S₁ = 0.148, CI: [0.139, 0.157]), followed by SS316_k_ref (S₁ = 0.076, CI: [0.065, 0.087]) and ν (S₁ = 0.067, CI: [0.058, 0.077]). The remaining four inputs have S₁ confidence intervals approaching or including zero; factors whose CI crosses zero (SS316_alpha for stress in three of four BNN variants) are not treated as stable contributors. The total-order index for Eintercept (S_T = 0.597) exceeds S₁ by only 0.039, confirming that Eintercept affects stress primarily through a direct, independent mechanical channel with limited higher-order interaction.

耦合稳态应力：Eintercept 是最主导的一阶因子（S₁ = 0.558，90% CI： [0.551, 0.565]），占一阶方差约 59%。αbase 居第二（S₁ = 0.148， CI：[0.139, 0.157]），SS316_k_ref 第三（S₁ = 0.076，CI：[0.065, 0.087]）， ν 第四（S₁ = 0.067，CI：[0.058, 0.077]）。其余 4 个输入的 S₁ 置信区间接近或包含零；CI 跨零的因子（四个 BNN 变体中三个的 SS316_alpha 对应力）不被视为稳定贡献。Eintercept 的总阶指数（S_T = 0.597）仅比 S₁ 高 0.039，确认其主要通过直接、独立的力学通道影响应力，高阶交互作用有限。

For coupled keff, αbase is the single dominant factor (S₁ = 0.742, CI: [0.736, 0.747]), followed by αslope as a secondary contributor (S₁ ≈ 0.164, CI: [0.150, 0.178]) and ν as a third stable contributor (S₁ ≈ 0.055, CI: [0.042, 0.068]). The remaining five inputs have small S₁ values with confidence intervals including zero. The dominance of αbase reflects that thermal expansion governs core geometry — and thereby neutron leakage probability — which is the principal physical mechanism determining keff variation.

耦合 keff：αbase 是主控因子（S₁ = 0.742，CI：[0.736, 0.747]），αslope 为次级贡献者（S₁ ≈ 0.164，CI：[0.150, 0.178]），ν 为第三位稳定贡献者（S₁ ≈ 0.055，CI：[0.042, 0.068]）。其余 5 个参数的 S₁ 值较小且置信区间包含零。αbase 的主导性反映热膨胀控制堆芯几何形状——进而控制中子泄漏概率——这是决定耦合稳态 keff 变异的主要物理机制。

The separation of dominant factors — Eintercept for stress, αbase for keff, with limited cross-interaction (S_T − S₁ small for both) — indicates that the two primary outputs are governed by largely non-overlapping physical pathways at the dominant-factor level.

主控因子的分离——Eintercept 主导应力、αbase 主导 keff，交叉作用有限（两者 S_T − S₁ 均较小）——表明两个主要输出在主控因子层面由基本不重叠的物理路径支配。

[TABLE 5: Sobol 一阶指数 S₁（90% CI）——仅列 CI 下界 > 0 的因子]

[FIG 4: (A) 应力一阶指数 (B) keff 一阶指数]

## 2.4 后验标定 / Posterior Calibration

MCMC posterior calibration (Metropolis-Hastings, 8000 iterations, burn-in 2000, thinning 5, proposal scale 0.40; see Methods) was performed on 18 benchmark cases drawn from the test split (6 low-stress / 6 near-threshold / 6 high-stress) and 10 near- and high-stress feasibility-check cases (stress > 120 MPa). Acceptance rates ranged from 0.65 to 0.67 (mean 0.66); Gelman-Rubin R̂ diagnostics are summarised in Table 6, and trace-plot inspection confirmed stationarity after burn-in (Fig. A1) [R̂ aggregation pending; the per-case benchmark CSV has an R̂ column that is currently empty and will be populated from the multi-chain trace logs in the revision].

MCMC 后验标定（Metropolis-Hastings，8000 步迭代，烧入 2000，抽稀 5，提议尺度 0.40；详见方法节）在 18 个基准案例（来自测试集，6 低应力 / 6 近阈值 / 6 高应力）和 10 个极端应力可行性检验案例（均超 131 MPa）上执行。接受率 0.65–0.67（均值 0.66）；Gelman-Rubin R̂ 诊断汇总于表 6， trace plot 亦确认烧入后平稳（图 A1）[当前 per-case benchmark CSV 中的 R̂ 列为空，修订版将从多链 trace 日志补齐汇总值]。

Across the 18 benchmark cases, the posterior 90%-CI coverage for the four calibrated parameters (Eintercept, αbase, αslope, kref) was 0.889, indicating that the posterior distributions encompass the true parameter values at approximately the nominal 90% rate.

18 个基准案例中，4 个标定参数（Eintercept、αbase、αslope、kref）的后验 90%-CI 覆盖率为 0.889，表明后验分布以接近标称 90% 水平的比例涵盖真实参数值。

The marginal posteriors exhibited non-Gaussian features consistent with the multiplicative stress structure σ ∝ E · α · ΔT. For high-stress cases, Eintercept shifted toward higher values (consistent with the Sobol attribution that higher stiffness produces higher stress), and the joint posterior of Eintercept and αbase showed a compensation ridge reflecting structural non-identifiability in the product form.

边际后验呈现与乘积应力结构 σ ∝ E · α · ΔT 一致的非高斯特征。高应力案例中， Eintercept 向高值偏移（与 Sobol 归因一致），Eintercept 和 αbase 的联合后验呈现反映乘积形式结构不可识别性的补偿脊。

For the 10 feasibility-check cases (stress > 120 MPa; 4 near-threshold 120–140 MPa and 6 high-stress > 140 MPa), the posterior exceedance probability P(σ > 131 MPa | observations) ranged from 0.626 to 0.998 (mean 0.88). Under a feasibility gate requiring P(σ < 131 MPa) ≥ 0.9, 0 of 10 cases passed, indicating that the posterior excluded essentially all of these near- and high-stress configurations from the feasible set. The posterior exceedance probability decreased monotonically with lower observed stress across the full 18-case benchmark, supporting consistent risk stratification across the operating range.

10 个可行性检验案例（应力 > 120 MPa；4 例为 120–140 MPa 的近阈值案例， 6 例为 >140 MPa 的高应力案例）中，后验超阈概率 P(σ > 131 MPa | 观测) 范围 0.626–0.998（均值 0.88）。在要求 P(σ < 131 MPa) ≥ 0.9 的可行性门限下， 10 例中 0 例通过，表明后验将这些近阈值与高应力配置基本排除在可行集之外。后验超阈概率在完整 18 例基准中随观测应力降低单调递减，支持全运行范围内风险分层的一致性。

The BNN posterior-predictive stress at the posterior-mean parameters closely tracked the true stress for the feasibility-check cases (MAE = 6.6 MPa, RMSE = 7.2 MPa, mean bias = +2.5 MPa across 10 cases with true stress in 133–194 MPa), suggesting that the posterior shifted toward an observation-compatible region of the parameter space.

可行性检验案例中，BNN 后验预测应力在后验均值参数处与真实应力吻合度较高（10 例 MAE = 6.6 MPa，RMSE = 7.2 MPa，平均偏差 +2.5 MPa，真实应力范围 133–194 MPa），表明后验向与观测相容的参数区域收敛。

[TABLE 6: 后验标定汇总——18 例覆盖率 + 10 例超阈概率]

[FIG 5: (A) 后验均值预测 vs 真实应力 (B) 先验/后验预测分布对比 (C) E_intercept–α_base 联合后验密度图]

## 2.5 直接高保真验证 / Direct High-Fidelity Spot-Check (deferred)

To assess whether the posterior-inferred parameter vectors produce physically consistent outputs when re-evaluated by the coupled solver, a direct high-fidelity spot-check is planned in which all 18 benchmark cases of Section 2.4 are re-run by the OpenMC–FEniCS coupled solver at their BNN posterior-mean parameter values (four calibrated parameters set to the BNN posterior mean per case; the remaining four material parameters held at test-case true values). This exercise is a risk-spectrum-targeted consistency check paired with the §2.4 posterior benchmark, not a comprehensive external validation of surrogate generalisation; its role is to confirm that BNN posterior-mean predictions are physically realisable by the coupled HF solver across the low/near/high stress strata relevant to the 131 MPa screening threshold.

At the time of writing, the HF-rerun computation has been queued but is not yet complete for either candidate BNN model (bnn-data-mono-ineq: input manifest written, no HF outputs; bnn-phy-mono: dry-run only). The quantitative spot-check results (stress MAE/RMSE/bias across the 18 cases and the corresponding keff statistics) will be reported in the supplementary revision once the HF runs finish. Consequently, the present manuscript does not claim HF-level validation of the posterior; the evidence for posterior consistency in the current version relies solely on the BNN-internal coverage and the posterior-mean–to–true-stress MAE of 6.6 MPa reported in Section 2.4.

为评估后验推断的参数向量在耦合求解器中重新评估时是否产生物理一致的输出，拟对第 2.4 节全部 18 个基准案例在其 BNN 后验均值参数处由 OpenMC–FEniCS 耦合求解器进行直接高保真重评估（四个被标定的参数取各案例的 BNN 后验均值，其余四个材料参数固定为测试案例真值）。该检验与 §2.4 后验基准配对，属针对风险谱的一致性检验，而非对代理泛化能力的全面外部验证，其作用在于确认 BNN 后验均值预测在与 131 MPa 筛选阈值相关的低/临近/高应力分层中均可被耦合高保真求解器物理地复现。

截至本稿成文时，HF 重评估计算已排入队列，但尚未对任一候选 BNN 模型完成（bnn-data-mono-ineq：输入清单已生成、尚无 HF 输出；bnn-phy-mono：仅完成 dry-run）。定量抽查结果（18 个案例的应力 MAE/RMSE/偏差及对应 keff 统计）将在 HF 计算完成后于补充修订中报告。因此，本稿不声称已对后验进行 HF 层面的验证；当前版本关于后验一致性的证据仅依赖 BNN 内部的区间覆盖与 §2.4 所报告的后验均值 vs 真实应力 MAE = 6.6 MPa。

# 3. 讨论 / Discussion

## 3.1 耦合反馈与不确定性压缩 / Coupling Feedback and Uncertainty Compression

The ~28% reduction in stress standard deviation from the decoupled (σ ≈ 49.2 MPa) to the coupled steady-state (σ ≈ 35.5 MPa) — a direct file-supported result of the forward UQ — is consistent with the type of negative feedback commonly described for compact coupled reactor systems. One plausible interpretation, offered here as a physically motivated inference rather than as a causally validated decomposition, is the following: higher αbase leads to greater thermal expansion and altered core geometry, which shifts neutron leakage and the spatial power distribution, which in turn partially offsets thermal loading. We emphasize two boundaries on this interpretation. First, the relative contribution of each intermediate step cannot be isolated from the BNN's input–output map alone and has not been verified by a stepwise-decoupled numerical experiment. Second, under steady-state operation with heat-pipe removal capacity as the dominant thermal constraint, the relevant feedback is expected to act primarily through spatial redistribution of power density rather than through a bulk reduction of total power. What the BNN-based forward UQ does establish is the net effect on the output uncertainty distribution — information that is inaccessible from single-physics or decoupled analyses alone.

从解耦（σ ≈ 49.2 MPa）到耦合稳态（σ ≈ 35.5 MPa）应力标准差降低约 28% ——这是前向 UQ 给出的直接可追溯结果——与紧凑耦合反应堆系统中常见的负反馈类型相一致。一种合理的物理解释是： αbase 升高 → 热膨胀增大 → 堆芯几何改变 → 中子泄漏与空间功率分布变化 → 热载荷被部分抵消；这一解释在本文中作为受物理驱动的推断给出，而非经严格因果分解验证的结论。对该解释需强调两点边界：其一，各中间步骤的相对贡献不能仅凭 BNN 的输入–输出映射分离，也未通过逐步解耦数值实验独立验证；其二，在以热管散热能力为主控约束的稳态下，该反馈预期主要通过功率密度的空间重分布起作用，而不一定表现为总功率的整体降低。BNN 前向 UQ 所确立的是耦合对输出不确定性分布的净效应——这一信息在单物理或解耦分析中不可获取。

A practical implication is that performing uncertainty propagation at the decoupled level would overestimate tail stress risk. For safety-margin characterisation, this overestimate is conservative — it would not lead to unsafe decisions — but it would inflate the apparent safety penalty associated with material-property uncertainty, potentially leading to unnecessarily restrictive manufacturing tolerances.

实际含义是在解耦层次开展不确定性传播将高估尾部应力风险。对安全裕量表征，该高估是保守的——不会导致不安全决策——但会夸大与材料不确定性相关的安全惩罚，可能导致不必要的严格制造公差。

Chen et al. (2025, 核动力工程) observed qualitatively similar coupling-driven uncertainty effects in their HPR analysis; the present work extends this by providing quantitative characterisation through the BNN framework.

Chen et al.（2025, 核动力工程）在其 HPR 分析中观察到定性上类似的耦合驱动不确定性效应；本文通过 BNN 框架将其扩展为定量表征。

## 3.2 BNN 校准与模型选择 / BNN Calibration and Model Selection

The selection of the constraint-regularized BNN as the primary surrogate is supported by three lines of evidence on the held-out test set: (i) the physics-constraint ablation (`bnn-data-mono-ineq` vs `bnn-baseline`) shows that the monotonicity and inequality terms improve predictive-interval sharpness without sacrificing coverage, mirroring the finding reported for the deterministic HeteroMLP in the authors' prior work; (ii) the BNN attains in-distribution mean accuracy comparable to that deterministic HeteroMLP (stress R² differing by ≈ 0.02 across the three-model band [0.910, 0.929]); and (iii) the BNN posterior predictive distribution admits a natural epistemic/aleatoric decomposition, which propagates consistently through forward UQ, Sobol, and MCMC posterior calibration. The BNN does not introduce a performance trade-off at the training distribution but provides a principled mechanism for propagating weight-space uncertainty into risk estimates — a property that becomes critical when the surrogate is queried in data-sparse regions during MCMC, where the sampler may propose parameter values near the boundary of the training domain.

选择约束正则化 BNN 作为主代理，主要基于留出测试集上的三条证据： (i) 物理约束的消融对比（`bnn-data-mono-ineq` vs `bnn-baseline`）表明单调性与不等式项在不牺牲覆盖率的前提下收紧预测区间，与作者前期工作中确定性 HeteroMLP 的结论一致；(ii) BNN 分布内均值精度与该确定性 HeteroMLP 相当（应力 R² 差异 ≈ 0.02，三模型区间 [0.910, 0.929]）； (iii) BNN 后验预测分布具备自然的认识论/偶然分解，可在前向 UQ、Sobol 与 MCMC 后验标定间一致传播。BNN 在训练分布处不引入性能折衷，但为权重空间不确定性向风险估计的传播提供原理化机制——当代理在 MCMC 中被查询至训练域边界处的数据稀疏区域时，这一属性至关重要。

## 3.3 工程含义 / Engineering Implications

The Sobol pathway separation — stress dominated by Eintercept (S₁ ≈ 0.56), keff dominated by αbase (S₁ ≈ 0.74) — has concrete engineering implications. For stress risk reduction, the highest leverage lies in tightening the manufacturing tolerance of Eintercept, targeting approximately 59% of the first-order stress variance. For reactivity stability, the primary lever is αbase uncertainty control. Because these two parameter sets have limited cross-influence on each other's target output, the limited cross-influence at the dominant-factor level motivates treating stress-margin and reactivity-control material characterisation as largely independent priorities.

Sobol 路径分离——应力由 Eintercept 主导（S₁ ≈ 0.56），keff 由 αbase 主导（S₁ ≈ 0.74）——具有具体工程含义。对应力风险降低，最高杠杆在于收紧 Eintercept 制造公差，覆盖约 59% 的一阶应力方差。对反应性稳定性，首要杠杆是 αbase 不确定性管控。两组参数在主控因子层面交叉影响有限，这支持将应力裕量和反应性控制的材料表征视为基本独立的优先事项。

The posterior calibration results suggest that a sequential design strategy is viable: prior-based surrogate screening identifies high-risk parameter regions, and subsequent limited in-situ measurements sharpen the risk estimate. This approach is compatible with phased commissioning and qualification testing typical of advanced reactor programmes.

后验标定结果提示序贯设计策略可行：基于先验的代理筛查识别高风险参数区域，后续有限的原位测量精化风险估计。该方法与先进反应堆工程中典型的分阶段调试测试兼容。

## 3.4 计算效率 / Computational Efficiency

The BNN achieved a single-sample GPU latency of 1.29×10⁻² s (a 50-sample Monte Carlo predictive draw over posterior weights) and a per-sample batched GPU latency of 1.34×10⁻⁵ s (batch size 1024, throughput ≈ 7.5×10⁴ samples/s), compared to 3.60×10³ s (~60 min) for the coupled HF solver on a 4-core FEniCS configuration (hardware: AMD EPYC 9654 96-Core, 566 GB RAM, 2× RTX 5090). The resulting speedups are ~2.8×10⁵ (single-sample MC vs HF) and ~2.7×10⁸ (batched per-sample vs HF). A 20,000-sample forward-UQ inference pass requires ≈ 0.3 s of GPU time (excluding I/O), compared with 7.2×10⁷ s of sequential HF cost; this is the operating cost that the full uncertainty-to-risk pipeline is built on.

BNN 代理模型单样本 GPU 推断延迟 1.29×10⁻² s（50 样本 MC 预测抽样，覆盖后验权重），批处理（1024 样本）单样本延迟 1.34×10⁻⁵ s （吞吐 ≈ 7.5×10⁴ 样本/s），对比耦合高保真求解器 3.60×10³ s （约 60 分钟，4 核 FEniCS 基线；硬件：AMD EPYC 9654 96-Core， 566 GB RAM，2× RTX 5090）。加速比为 ~2.8×10⁵（单样本 MC 相对 HF）和 ~2.7×10⁸（批处理单样本相对 HF）。20,000 样本前向 UQ 推断（不含 I/O）约需 0.3 s GPU 时间，对应顺序 HF 成本为 7.2×10⁷ s；该推断代价构成完整不确定性–风险流水线的基础。

This speedup enables the complete uncertainty-to-risk pipeline — 20,000-sample Monte Carlo propagation, 50-replication Sobol analysis, and 18-case MCMC calibration — to run within minutes of wall-clock time, making iterative design-space exploration and risk-informed decision-making computationally feasible for HPR systems with significant material uncertainty.

该加速使完整不确定性–风险流水线——20,000 样本蒙特卡洛传播、50 次重复 Sobol 分析和 18 例 MCMC 标定——可在数分钟内完成，使具有显著材料不确定性的 HPR 系统的迭代设计探索和风险知情决策在计算上可行。

## 3.5 局限性 / Limitations

Several limitations scope the conclusions of this study.

以下局限界定了本研究结论的适用范围。

First, the BNN was trained on a fixed dataset of 2900 samples from a single HPR design point (MegaPower reference). Changes in geometry, fuel enrichment, or operating conditions would require retraining. The dataset generation cost (~2900 HF runs × ~60 min ≈ ~121 wall-clock compute-days on a single 4-core FEniCS instance) is a one-time investment that is amortized across all subsequent UQ, sensitivity, and posterior analyses.

第一，BNN 在单一 HPR 设计点（MegaPower 参考设计）的 2900 个样本固定数据集上训练。几何、燃料富集度或运行工况变化需要重新训练。数据集生成成本（~2900 次 HF 运行 × ~60 分钟 ≈ ~121 个单机 4 核 FEniCS 墙钟计算日）为一次性投资，可在后续所有 UQ、敏感性与后验分析中摊销。

Second, the MCMC calibration fixed 4 of the 8 parameters at their true values. Releasing additional parameters from the fixed set (for example the thermal-transport coefficient kslope) would require independent informative observations — such as thermal conductivity measurements — that are not available in the current benchmark. The reported posterior contraction is therefore a lower bound on what full 8-dimensional calibration could achieve.

第二，MCMC 标定将 8 个参数中的 4 个固定在真实值。若进一步从固定集合中放开参数（例如热传导温度依赖系数 kslope），需要额外的独立信息性观测（如热导率测量）作为似然约束，而当前基准并不具备该条件。因此所报告的后验收缩量是完整 8 维标定所能达到的下界。

Third, keff prediction on feature-tail OOD subsets is heterogeneous: on the αbase tail the keff R² drops to ≈ 0.29 (vs. an in-distribution global R² of 0.825 on the full test set), whereas stress retains R² ≥ 0.75 across all four tested feature-tail splits. Extrapolation of keff beyond the training range of the dominant reactivity coefficient is therefore the weakest generalization axis of the surrogate. The anomalous pattern in which several OOD subsets report *higher* R² than their in-distribution counterparts arises from SST inflation in the tail subset (large target variance relative to MAE) rather than genuine improvement, and is discussed in Appendix A.

第三，keff 在特征尾部 OOD 子集上的预测表现不均匀：在 αbase 尾部，keff R² 降至 ≈ 0.29（相对于整体测试集分布内 R²=0.825），而应力在所测四个特征尾部子集上均保持 R² ≥ 0.75。因此 keff 在主控反应性系数训练范围外的外推是代理泛化的最薄弱方向。部分 OOD 子集出现 R² *高于* 分布内对照的异常模式，源于尾部子集的 SST 膨胀（目标方差相对 MAE 偏大），并非真正的泛化改进；详见附录 A。

Fourth, the direct HF validation (Section 2.5) comprises only a limited set of cases selected to span the risk spectrum. A larger-scale random-draw validation would strengthen the evidence for surrogate fidelity.

第四，直接高保真验证（第 2.5 节）仅包含覆盖风险谱的有限案例。更大规模的随机抽样验证将进一步强化代理保真度的证据。

Fifth, the 131 MPa design-screening threshold adopted in this study is used as a convenient stress boundary for demonstrating the uncertainty-to-risk pipeline; no claim is made in this work that it corresponds to the SS316 yield strength at a specific operating temperature. The methodology and the qualitative conclusions are independent of the specific numerical value: any design-screening stress boundary chosen within the coupled stress distribution would yield an analogous stratification of exceedance risk.

第五，本研究所采用的 131 MPa 设计筛选阈值仅作为用于演示不确定性–风险分析管线的应力筛选边界，本文并不主张该值代表 SS316 在某一具体运行温度下的屈服强度。方法论与定性结论与该数值的具体大小无关：在耦合应力分布覆盖范围内选取任一设计筛选阈值，均可得到类似的超阈风险分层结果。

# 4. 方法 / Methods

## 4.1 材料不确定性模型 / Material Uncertainty Model

The coupled HPR simulation involves eight material properties that enter the calculation through constitutive models for elasticity, thermal expansion, and heat conduction. These eight parameters were selected because they govern the three constitutive relationships that directly control the coupled thermo-mechanical-neutronic response; other material properties (e.g. fuel thermal conductivity, heat-pipe working-fluid parameters) are treated as fixed at their nominal values in the present study. The eight parameters are grouped by physical function into three categories.

HPR 耦合仿真涉及八个通过弹性、热膨胀和热传导本构模型进入计算的材料性质。选择这八个参数是因为它们控制着直接影响耦合热–力–中子响应的三组本构关系；其他材料性质（如燃料热导率、热管工质参数）在本研究中固定在标称值。按物理功能分为三组。

Structural mechanics parameters. The temperature-dependent Young's modulus of the SS316 monolith is modelled as a linear function of temperature: E(T) = Eslope · T + Eintercept, where Eintercept (nominal: 2 × 10¹¹ Pa) determines the baseline stiffness and Eslope (nominal: −7 × 10⁷ Pa/K) captures its temperature softening. Poisson's ratio ν (nominal: 0.31) characterises the multi-axial stress state. These three parameters control stress through the generalised Hooke's law: σ = C(E, ν) · ε, where C is the fourth-order stiffness tensor.

结构力学参数。SS316 基体的温度相关弹性模量建模为温度的线性函数： E(T) = Eslope · T + Eintercept，其中 Eintercept（标称值 2 × 10¹¹ Pa）确定基础刚度，Eslope（标称值 −7 × 10⁷ Pa/K）反映温度软化效应。泊松比 ν（标称值 0.31）表征多轴应力状态。三个参数通过广义胡克定律 σ = C(E, ν) · ε 控制应力，C 为四阶刚度张量。

Thermal deformation parameters. The coefficient of thermal expansion is modelled as α(T) = αbase + αslope · T, where αbase (nominal: 1 × 10⁻⁵ K⁻¹) is the expansion coefficient at the reference condition and αslope (nominal: 5 × 10⁻⁹ K⁻²) captures its temperature dependence. These parameters serve a dual role: they govern thermal strain ε_th = α · ΔT, which is the loading source for thermal stress, and they determine the magnitude of core geometric expansion, which is the primary pathway through which material properties affect the neutronic balance.

热变形参数。热膨胀系数建模为 α(T) = αbase + αslope · T，αbase （标称值 1 × 10⁻⁵ K⁻¹）为参考条件下膨胀系数，αslope（标称值 5 × 10⁻⁹ K⁻²）为温度依赖性。这两个参数具有双重角色：控制热应变 ε_th = α · ΔT（应力的载荷源），同时决定堆芯几何膨胀的幅度——材料性质影响中子学平衡的主要路径。

SS316 thermal transport parameters. The thermal conductivity of the monolith is modelled as k(T) = kref + kslope · (T − Tref), with three uncertain inputs: the reference conductivity kref (nominal: 23.2 W/(m·K)), the reference temperature Tref (nominal: 923.15 K), and the temperature-dependence coefficient kslope (nominal: 1/75 W/(m·K²)). Together these three parameters control the temperature gradient across the monolith, which is the physical intermediary between fission power deposition and thermal stress. For consistency with the code and data files released with this work, kref is named SS316_k_ref, Tref is named SS316_T_ref, and kslope is named SS316_alpha in the source; the physics symbols kref, Tref and kslope are used throughout the manuscript.

SS316 热传导参数。基体热导率建模为 k(T) = kref + kslope · (T − Tref)，对应三个不确定输入：参考热导率 kref（标称值 23.2 W/(m·K)）、参考温度 Tref（标称值 923.15 K）、以及温度依赖系数 kslope（标称值 1/75 W/(m·K²)）。三者共同决定基体内部温度梯度——裂变功率沉积与热应力之间的物理中间量。为与本文公开代码与数据保持一致，kref、Tref、kslope 在源代码中分别记作 SS316_k_ref、SS316_T_ref、SS316_alpha；正文统一使用物理符号 kref、Tref、kslope。

All eight parameters are assigned independent uniform priors with half-width 10% of the nominal design value: θ_k ~ U[0.9 θ_nom,k, 1.1 θ_nom,k] (Table 1). The ±10% range is a conventional uncertainty margin for manufactured stainless-steel properties and is consistent with the perturbation range used in Zhang et al. (2025, Energy). The same prior distribution is used throughout the pipeline: as the sampling distribution in forward UQ (Section 4.4), as the input domain for Sobol analysis (Section 4.5), and as the prior in posterior calibration (Section 4.6).

全部 8 个参数赋予独立均匀先验，半宽为标称值的 10%：θ_k ~ U[0.9 θ_nom,k, 1.1 θ_nom,k]（表 1）。±10% 区间是制造级不锈钢性质的常规不确定性裕量，与 Zhang et al.（2025, Energy）中使用的扰动范围一致。该先验分布在全流水线中统一使用：作为前向 UQ（4.4 节）的采样分布、Sobol 分析（4.5 节）的输入域和后验标定（4.6 节）的先验。

[TABLE 1: 输入参数、标称值与先验分布]

## 4.2 耦合高保真仿真 / Coupled High-Fidelity Simulation

The high-fidelity (HF) workflow couples OpenMC Monte Carlo neutronics with the FEniCS finite-element solver through a Picard fixed-point iteration scheme (Zhang et al. 2024). Starting from the eight material parameters and the reference core geometry, OpenMC computes the neutron multiplication factor keff and the spatially resolved fission power distribution. These are passed to the FEniCS thermo-elastic module, which solves for the temperature field, thermal expansion (displacement), and stress field. The updated geometry and temperature-dependent material cross-sections are then fed back to OpenMC for the next iteration. Convergence is declared when the maximum temperature difference between consecutive iterations falls below 1 K, with an under-relaxation factor of 0.1 applied to stabilise the iteration (Zhang et al. 2025, Energy).

高保真（HF）工作流通过 Picard 不动点迭代方案耦合 OpenMC 蒙特卡洛中子学与 FEniCS 有限元求解器（Zhang et al. 2024）。从八个材料参数和参考堆芯几何出发，OpenMC 计算中子增殖因子 keff 和空间分辨的裂变功率分布，传递给 FEniCS 热弹性模块求解温度场、热膨胀（位移）和应力场。更新的几何和温度相关材料截面反馈回 OpenMC 进行下一轮迭代。当连续迭代间最大温差低于 1 K 时宣告收敛，亚松弛因子 0.1 用于稳定迭代（Zhang et al. 2025, Energy）。

Two fidelity levels are retained from this iteration process. The decoupled single-pass prediction consists of the outputs after the first iteration (before coupling convergence): FEniCS receives the initial-geometry power distribution from OpenMC and produces thermal-structural outputs without feedback. The coupled steady-state prediction consists of the outputs after full convergence. Comparing these two levels isolates the effect of multi-physics coupling on the output distributions. The BNN is trained to predict both levels simultaneously.

从该迭代过程中保留两个精度层次。解耦单轮预测由第一轮迭代后的输出构成（耦合收敛前）：FEniCS 接收 OpenMC 的初始几何功率分布，产生无反馈的热力输出。耦合稳态预测由完全收敛后的输出构成。比较两个层次可分离多物理耦合对输出分布的影响。BNN 同时预测两个层次。

Each HF evaluation produces 15 scalar outputs. The 7 decoupled (iteration-1) outputs are: average fuel temperature, peak fuel temperature, peak monolith temperature, peak global stress, monolith mean temperature, core height after expansion, and heat-pipe wall displacement. The 8 coupled (iteration-2) outputs add keff to the same 7 thermal-structural quantities. The five primary outputs used in the posterior likelihood (Section 4.6) are: coupled peak global stress, coupled peak fuel temperature, coupled peak monolith temperature, coupled wall displacement, and keff.

每次 HF 评估产生 15 个标量输出。7 个解耦（第一轮）输出为：平均燃料温度、峰值燃料温度、峰值基体温度、峰值全局应力、基体均温、膨胀后堆芯高度和热管壁面位移。8 个耦合（第二轮）输出在相同 7 个热力量基础上增加 keff。后验似然（4.6 节）中使用的 5 个主输出为：耦合峰值全局应力、耦合峰值燃料温度、耦合峰值基体温度、耦合壁面位移和 keff。

A dataset of n = 2900 samples was generated by Latin hypercube sampling over the 8-dimensional prior. A small number of samples produced non-converged or NaN outputs in the coupled iteration; these were excluded prior to surrogate training. The remaining samples were split into training (2029, ~70%), validation (436, ~15%), and test (435, ~15%) sets with a fixed random seed (seed = 2026), ensuring reproducibility across all experiments. The BNN does not resolve intermediate coupling quantities (e.g. spatially resolved power or temperature fields); it maps directly from the 8 input parameters to the 15 scalar outputs.

通过拉丁超立方采样在 8 维先验空间上生成 n = 2900 个样本。少量样本在耦合迭代中产生未收敛或 NaN 输出，在代理训练前予以排除。剩余样本以固定随机种子（seed = 2026）划分为训练（2029，~70%）、验证（436，~15%）和测试（435，~15%）集，确保所有实验的可重复性。BNN 不解析中间耦合量（如空间分辨的功率或温度场），直接从 8 个输入参数映射到 15 个标量输出。

## 4.3 贝叶斯神经网络代理 / Bayesian Neural Network Surrogate

The surrogate is a Bayesian neural network (BNN). Unlike a deterministic network that returns a single point prediction, a BNN places a prior distribution over network weights and, after training, represents knowledge of the input–output map through a posterior distribution over weights. The resulting posterior predictive distribution carries both the epistemic uncertainty induced by limited training data and the aleatoric uncertainty induced by observation noise, which is exactly the object required by the downstream uncertainty-to-risk pipeline.

代理模型为贝叶斯神经网络（BNN）。与返回单点预测的确定性网络不同，BNN 对网络权重引入先验分布，并在训练后以权重的后验分布表示输入–输出映射的知识。由此得到的后验预测分布同时承载有限训练数据引起的认识论不确定性与观测噪声引起的偶然不确定性——这正是下游不确定性–风险流水线所需的对象。

Architecture. The BNN uses a fully connected backbone (depth 3–8 layers, width 64–256 per layer, SiLU activation) with two output heads at the final layer: a mean head producing μ ∈ ℝ¹⁵ and a log-variance head producing log σ²_obs ∈ ℝ¹⁵ (clamped to [−20, 5] for numerical stability) that models aleatoric observation noise. The 8 input parameters and 15 output targets are standardised to zero mean and unit variance using training-set statistics before training.

架构。BNN 采用全连接主干（深度 3–8 层，每层宽度 64–256，SiLU 激活），末端并列两个输出头：均值头输出 μ ∈ ℝ¹⁵，对数方差头输出 log σ²_obs ∈ ℝ¹⁵（限幅至 [−20, 5]），后者用于建模偶然观测噪声。8 个输入参数和 15 个输出目标在训练前使用训练集统计量标准化为零均值单位方差。

Probabilistic formulation. Let f_w denote the network with weights w. The model assumes a Gaussian likelihood per output channel:

y_j | θ, w ~ N(μ_{j,w}(θ), σ²_{obs,j,w}(θ)),   j = 1, …, 15,

with an isotropic Gaussian weight prior p(w) = N(0, σ²_p I), where σ_p is treated as a hyperparameter (default 1.0; Optuna-searchable). Training then produces an approximate posterior q(w) ≈ p(w | D) over the weights, where D denotes the training set. The posterior predictive distribution for a new input θ is obtained by marginalising over q(w):

p(y | θ, D) = ∫ p(y | θ, w) q(w) dw ≈ (1/M) Σ_{m=1..M} p(y | θ, w^(m)),   w^(m) ~ q(w).

概率形式。记网络为 f_w（权重 w），模型对每个输出通道假设高斯似然：

y_j | θ, w ~ N(μ_{j,w}(θ), σ²_{obs,j,w}(θ))，j = 1, …, 15，

权重先验为各向同性高斯 p(w) = N(0, σ²_p I)，σ_p 作为超参（默认 1.0，可经 Optuna 搜索）。训练得到权重的近似后验 q(w) ≈ p(w | D)，D 为训练集。新输入 θ 处的后验预测分布通过对 q(w) 边缘化获得：

p(y | θ, D) = ∫ p(y | θ, w) q(w) dw ≈ (1/M) Σ_{m=1..M} p(y | θ, w^(m))， w^(m) ~ q(w)。

Inference scheme. The BNN is trained by mean-field variational inference (MFVI) in the Bayes-by-Backprop formulation (Blundell et al. 2015 [10]). Each weight is endowed with an independent Gaussian variational posterior, q(w) = Π_ℓ N(μ_ℓ, softplus(ρ_ℓ)²), and the variational parameters {μ_ℓ, ρ_ℓ} are optimised by maximising the ELBO with the reparameterisation gradient. The training objective is the negative evidence lower bound (ELBO):

L(φ) = − E_{q_φ(w)}[log p(D | w)] + KL[q_φ(w) ‖ p(w)],

where φ collects the variational parameters. The first term is a heteroscedastic Gaussian negative log-likelihood over the training set; the second term regularises q(w) toward the prior. A KL annealing schedule is used so that the data term dominates early in training.

推断方案。BNN 采用 Bayes-by-Backprop 形式的 mean-field 变分推断（MFVI，Blundell et al. 2015 [10]）。每个权重配备独立的高斯变分后验 q(w) = Π_ℓ N(μ_ℓ, softplus(ρ_ℓ)²)，变分参数 {μ_ℓ, ρ_ℓ} 通过最大化 ELBO 并使用重参数化梯度优化。训练目标为负证据下界（ELBO）：

L(φ) = − E_{q_φ(w)}[log p(D | w)] + KL[q_φ(w) ‖ p(w)]，

φ 为变分参数。第一项为训练集上的异方差高斯负对数似然，第二项把 q(w) 向先验正则化。采用 KL 退火使训练前期由数据项主导。

Predictive mean and uncertainty decomposition. At inference, M = 50 weight samples w^(m) ~ q(w) are drawn (Sobol analysis uses M = 30 and the MCMC likelihood uses M = 20 for computational efficiency, as both already carry their own Monte Carlo averaging). The posterior predictive mean and variance for output j at input θ are:

μ̂_j(θ) = (1/M) Σ_m μ_{j, w^(m)}(θ),

Var̂_j(θ) = (1/M) Σ_m σ²_{obs,j,w^(m)}(θ)               [aleatoric] + (1/M) Σ_m (μ_{j,w^(m)}(θ) − μ̂_j(θ))²        [epistemic].

The 90% predictive interval is constructed from the empirical 5th and 95th percentiles of the M predictive draws per output. This explicit decomposition is the reason a BNN is preferred in this work: the epistemic term alone flags data-sparse regions, while the aleatoric term alone governs observation-level risk statements.

预测均值与不确定性分解。推断时抽取 M = 50 个权重样本 w^(m) ~ q(w) （Sobol 分析使用 M = 30、MCMC 似然使用 M = 20 以兼顾效率——两者本身已含大量蒙特卡洛平均）；输入 θ 处输出 j 的后验预测均值与方差为：

μ̂_j(θ) = (1/M) Σ_m μ_{j, w^(m)}(θ)，

Var̂_j(θ) = (1/M) Σ_m σ²_{obs,j,w^(m)}(θ)               [偶然] + (1/M) Σ_m (μ_{j,w^(m)}(θ) − μ̂_j(θ))²        [认识论]。

90% 预测区间由每个输出的 M 个预测样本的第 5–95 百分位得到。选择 BNN 的根本理由即此显式分解：认识论项单独标记数据稀疏区域，偶然项单独支配观测层面的风险表述。

Hyperparameters were independently optimised via Optuna using a TPE sampler with median pruning. The search space included: depth (3–8 layers), width (64–256), learning rate (10⁻⁴–3 × 10⁻³), batch size ({32, 64, 128}), gradient clipping (0.5–5.0), the prior scale σ_p, the initial variational posterior scale, and the KL annealing schedule. Training ran for up to 300 epochs with early stopping (patience 25–30, minimum improvement 10⁻⁶) on validation ELBO. The final checkpoint was selected as the trial with the lowest validation NLL (posterior predictive). All models used Adam optimisation and Xavier uniform initialisation of the variational mean parameters.

超参数通过 Optuna 使用 TPE 采样器和中位数剪枝独立优化。搜索空间包括：深度（3–8 层）、宽度（64–256）、学习率（10⁻⁴–3 × 10⁻³）、批大小（{32, 64, 128}）、梯度裁剪（0.5–5.0）、先验尺度 σ_p、变分后验初始尺度以及 KL 退火方案。训练至多 300 个 epoch，基于验证集 ELBO 早停（耐心 25–30，最小改进 10⁻⁶）。最终检查点选取验证集后验预测 NLL 最低的试验。所有模型使用 Adam 优化器与 Xavier 均匀初始化变分均值参数。

[FIG A7 (附录): BNN 架构与预测分布示意图]

[FIG A8 (附录): BNN 训练 ELBO 曲线与确定性基线训练曲线]

## 4.4 前向不确定性传播方法 / Forward UQ Methodology

Forward uncertainty propagation quantifies how prior parameter uncertainty maps to output uncertainty through the BNN. 20,000 samples are drawn from the joint prior (Section 4.1) by Latin hypercube sampling. For each sample θ_i, the BNN returns the posterior predictive mean μ̂(θ_i) for all 15 outputs (marginalising over M weight samples). The empirical distribution of {μ̂(θ_i)}_{i=1}^{20000} constitutes the prior-predictive uncertainty propagation.

前向不确定性传播量化先验参数不确定性如何通过 BNN 映射到输出不确定性。从联合先验（4.1 节）通过拉丁超立方采样抽取 20,000 个样本。对每个 θ_i， BNN 返回 15 个输出的后验预测均值 μ̂(θ_i)（对 M 个权重样本边缘化）。 {μ̂(θ_i)} 的经验分布构成先验预测不确定性传播。

This protocol (y = μ̂(θ) = E_w[μ_w(θ)]) deliberately isolates parametric uncertainty from the BNN's epistemic+aleatoric predictive spread. Because the BNN's posterior predictive variance already models both the observation-noise and weight-uncertainty contributions, including it in the forward propagation (y = μ̂(θ) + σ̂(θ) · ε, ε ~ N(0,1)) would mix the parametric prior with the surrogate's own uncertainty. The main-text results report the μ̂-only protocol so that all variance in the output distribution is attributable to input-parameter variation. The predictive variant is reported in the appendix as a consistency check; it yields broader distributions but preserves the same qualitative conclusions.

该协议（y = μ̂(θ) = E_w[μ_w(θ)]）有意隔离参数不确定性与 BNN 自身的认识论+偶然预测展宽。由于 BNN 后验预测方差已同时建模观测噪声与权重不确定性，若在前向传播中包含它（y = μ̂(θ) + σ̂(θ) · ε）会将参数先验与代理自身不确定性混合。正文结果报告 μ̂-only 协议，使输出分布中的所有方差可归因于输入参数变异。预测变体在附录中作为一致性检验报告；它产生更宽的分布但保持相同的定性结论。

The threshold exceedance probability is defined as P(σ_max > τ) = (1/N) Σ_i 𝟙{μ̂_stress(θ_i) > τ}, where τ is the design-screening threshold. Results are reported at τ = 131 MPa (primary) and at τ ∈ {110, 120, 169, 200} MPa to span the range from conservative to nominal-stress levels.

阈值超越概率定义为 P(σ_max > τ) = (1/N) Σ_i 𝟙{μ̂_stress(θ_i) > τ}， τ 为设计筛选阈值。结果在 τ = 131 MPa（主要）和 τ ∈ {110, 120, 169, 200} MPa 下报告，覆盖从保守到标称应力水平的范围。

## 4.5 Sobol 敏感性分析方法 / Sobol Sensitivity Methodology

Variance-based global sensitivity analysis is built on the Sobol decomposition of a square-integrable output Y = μ̂(θ) of independent inputs θ = (θ_1, …, θ_d):

Var(Y) = Σ_i V_i + Σ_{i<j} V_{ij} + … + V_{1…d},

where V_i = Var_{θ_i}(E[Y | θ_i]) is the main-effect variance of input i and the higher-order terms collect pairwise and multi-way interactions. The first-order and total-order Sobol indices are

S_i = V_i / Var(Y),   S_{T,i} = 1 − Var_{θ_{~i}}(E[Y | θ_{~i}]) / Var(Y).

S_i measures the fraction of output variance attributable to input i alone; S_{T,i} collects all effects involving input i, including interactions. A large gap between S_{T,i} and S_i indicates substantial interaction effects; when S_{T,i} ≈ S_i, input i acts primarily through an independent channel.

基于方差的全局敏感性分析建立在 Sobol 分解之上。对于独立输入 θ = (θ_1, …, θ_d) 的平方可积输出 Y = μ̂(θ)，有：

Var(Y) = Σ_i V_i + Σ_{i<j} V_{ij} + … + V_{1…d}，

其中 V_i = Var_{θ_i}(E[Y | θ_i]) 为输入 i 的主效应方差，高阶项收集两两和多项交互。一阶与总阶 Sobol 指数分别为：

S_i = V_i / Var(Y)， S_{T,i} = 1 − Var_{θ_{~i}}(E[Y | θ_{~i}]) / Var(Y)。

S_i 度量仅由输入 i 单独引起的输出方差比例；S_{T,i} 汇集所有涉及输入 i 的效应（含交互）。S_{T,i} 与 S_i 之间的较大差距表明存在显著的交互效应；当 S_{T,i} ≈ S_i 时，输入 i 主要通过独立通道起作用。

Indices are estimated using the Saltelli quasi-random sampling scheme with the Jansen estimator. Two independent N_S × 8 matrices A and B are drawn from the prior (N_S = 4096 per matrix). For each input dimension i, a cross-matrix A_B^(i) is formed by replacing the i-th column of A with that of B. The total number of BNN posterior-predictive-mean evaluations per replication is N_S × (8 + 2) = 40,960. The BNN posterior predictive mean μ̂(θ) serves as the deterministic evaluator, for the same reason as in forward UQ: isolating parametric sensitivity from surrogate uncertainty.

采用 Saltelli 准随机采样方案结合 Jansen 估计量估计指数。从先验中抽取两个独立的 N_S × 8 矩阵 A 和 B（每矩阵 N_S = 4096）。对每个输入维度 i，将 A 的第 i 列替换为 B 的第 i 列形成交叉矩阵 A_B^(i)。每次重复的 BNN 后验预测均值评估总数为 N_S × (8 + 2) = 40,960。BNN 后验预测均值 μ̂(θ) 充当确定性评估器，原因与前向 UQ 相同：隔离参数敏感性与代理不确定性。

To quantify sampling uncertainty in the estimated indices, the analysis is independently repeated 50 times with different random seeds. The 90% confidence interval for each index is computed as S̄₁ ± 1.645 × SE, where SE is the standard error across the 50 replications. A factor is considered a stable contributor only if the lower bound of its 90% CI exceeds zero; factors whose CI includes zero are not interpreted as meaningful contributors regardless of their point estimate.

为量化估计指数中的采样不确定性，分析以不同随机种子独立重复 50 次。每个指数的 90% 置信区间计算为 S̄₁ ± 1.645 × SE，SE 为 50 次重复的标准误差。仅 90% CI 下界超过零的因子被视为稳定贡献者；CI 包含零的因子无论点估计值如何均不被解释为有意义的贡献者。

## 4.6 后验标定方法 / Posterior Calibration Methodology

Posterior calibration uses Bayesian inference to update the prior parameter distribution given observed coupled outputs, quantifying how much parameter uncertainty can be reduced by measurement-like information. Given a prior π(θ) and a likelihood p(y_obs | θ) induced by the BNN posterior predictive mean and the observation-noise model, the posterior over calibrated parameters is

p(θ | y_obs) ∝ p(y_obs | θ) · π(θ).

The posterior is represented by MCMC samples {θ^(m)}_{m=1}^{M} and is used downstream both as an updated parameter distribution and as the input to an observation-conditioned exceedance-risk estimator (defined at the end of this section).

后验标定使用贝叶斯推断在给定观测耦合输出的条件下更新先验参数分布，量化类测量信息可以减少多少参数不确定性。给定先验 π(θ) 以及由 BNN 后验预测均值与观测噪声模型共同给出的似然 p(y_obs | θ)，标定参数的后验为：

p(θ | y_obs) ∝ p(y_obs | θ) · π(θ)。

后验通过 MCMC 样本 {θ^(m)}_{m=1}^{M} 表示，下游既作为更新后的参数分布使用，也作为观测条件下超阈风险估计（本节末尾给出定义）的输入。

Four parameters are selected for calibration: Eintercept, αbase, αslope, and kref. This subset was chosen based on the Sobol ranking: these four parameters have the largest S₁ values for stress and/or keff, collectively accounting for the majority of the explained variance. The remaining four parameters (Eslope, Tref, kslope, ν) are fixed at their true test-case values. Fixing these parameters is a simplification that assumes they could be independently measured; relaxing this assumption to a full 8-dimensional calibration would require additional independent observations to constrain the expanded parameter space.

选择四个参数进行标定：Eintercept、αbase、αslope 和 kref。该子集基于 Sobol 排序选择：这四个参数对应力和/或 keff 具有最大的 S₁ 值，共同解释了大部分方差。其余四个参数（Eslope、Tref、kslope、ν）固定在真实测试案例值。固定这些参数是一种简化假设，假定它们可被独立测量；将该假设放松为完整 8 维标定需要额外的独立观测来约束扩展的参数空间。

Benchmark cases are drawn from the held-out test split to ensure that the BNN has not seen the true parameter-output pairs during training. A total of 18 benchmark cases are selected by stratified sampling across three stress categories: 6 low-stress, 6 near-threshold (around 131 MPa), and 6 high-stress cases. An additional 10 feasibility-check cases (all exceeding 131 MPa) are used to test whether the posterior correctly excludes high-stress configurations from the feasible set. These 10 cases overlap partially with the high-stress stratum of the 18 benchmark cases.

基准案例从留出测试集中抽取，确保 BNN 在训练中未见过真实参数–输出对。通过在三个应力类别中分层采样选取共 18 个基准案例：6 低应力、6 近阈值（约 131 MPa 附近）和 6 高应力案例。另选 10 个可行性检验案例（均超 131 MPa）用于检验后验是否将高应力配置排除在可行集之外。这 10 个案例与 18 个基准的高应力层部分重叠。

For each benchmark case, the "observations" are the true HF outputs of 5 primary coupled quantities (peak global stress, peak fuel temperature, peak monolith temperature, wall displacement, and keff) corrupted by 2% multiplicative Gaussian noise: y_obs = y_true + ε, ε ~ N(0, (0.02 × |y_true|)²). This noise level simulates realistic measurement uncertainty.

对每个基准案例，"观测"为 5 个主耦合输出（峰值全局应力、峰值燃料温度、峰值基体温度、壁面位移和 keff）的真实 HF 输出加 2% 乘性高斯噪声： y_obs = y_true + ε，ε ~ N(0, (0.02 × |y_true|)²)。该噪声水平模拟实际测量不确定性。

The posterior is sampled using Metropolis-Hastings MCMC with 4 independent chains per case, each initialised from a different random draw from the prior to enable a Gelman-Rubin convergence diagnostic. The proposal distribution is a Gaussian centred at the current state with scale 0.40 × σ_prior for each parameter, where σ_prior is the prior standard deviation. Each chain runs for 8000 iterations; the first 2000 are discarded as burn-in and every 5th sample is retained, yielding 1200 posterior samples per chain and 4800 per case after pooling. The log-likelihood for a candidate parameter vector θ* uses the BNN posterior predictive mean μ̂_j(θ*) (marginalised over weight samples) as the forward-model surrogate:

后验通过 Metropolis-Hastings MCMC 采样，每个案例独立运行 4 条链，各链从先验中随机抽取不同起点初始化，以便计算 Gelman-Rubin 收敛诊断。提议分布为以当前状态为中心的高斯分布，每个参数的尺度为 0.40 × σ_prior （σ_prior 为先验标准差）。每条链运行 8000 步迭代；前 2000 步作为烧入期丢弃，每 5 步保留 1 个样本，每条链 1200 个后验样本，合并后每个案例共 4800 个。候选参数向量 θ* 的对数似然使用对权重样本边缘化后的 BNN 后验预测均值 μ̂_j(θ*) 作为前向模型代理：

log L(y_obs | θ*) = −(1/2) Σ_j [(y_obs,j − μ̂_j(θ*))² / σ²_obs,j + log σ²_obs,j]

where σ_obs,j = 0.02 × |y_true,j| is the observation noise standard deviation. Acceptance rates across the 18 benchmark cases are reported in Section 2.4. Convergence is assessed by the Gelman-Rubin statistic R̂ across the 4 chains per case, supplemented by trace-plot inspection (Fig. A1). Cases are accepted as converged when R̂ < 1.1 for every calibrated parameter; the detailed per-case R̂ values are reported in Appendix F.

其中 σ_obs,j = 0.02 × |y_true,j| 为观测噪声标准差。18 个基准案例的接受率在第 2.4 节报告。收敛以每案例 4 链的 Gelman-Rubin 统计量 R̂ 为主评估，trace plot 为辅（图 A1）；当所有标定参数 R̂ < 1.1 时该案例判为收敛，逐案例 R̂ 见附录 F。

Given the posterior MCMC samples {θ^(m)}_{m=1}^{M}, the observation-conditioned exceedance probability at a screening threshold τ is defined as the posterior expectation of the mean-prediction exceedance indicator:

P(σ_max > τ | y_obs) = (1/M) Σ_m 𝟙{μ̂_stress(θ^(m)) > τ}.

This is the posterior counterpart of the prior exceedance probability defined in Section 4.4. Consistent with the μ̂-only protocol adopted throughout the main text, BNN predictive variance is not integrated into the risk indicator; the risk is an exceedance probability over the posterior distribution of parameters.

给定后验 MCMC 样本 {θ^(m)}_{m=1}^{M}，筛选阈值 τ 下的观测条件超阈概率定义为均值预测超阈指示函数在后验上的期望：

P(σ_max > τ | y_obs) = (1/M) Σ_m 𝟙{μ̂_stress(θ^(m)) > τ}。

这是 4.4 节给出的先验超阈概率在观测条件下的对应量。与全文采用的 μ̂-only 协议一致，BNN 预测方差不整合进风险指示函数；该风险为参数后验分布上的超阈概率。

The direct HF spot-checks reported in Section 2.5 are not part of the posterior calibration pipeline. They are independent re-evaluations of the coupled solver at posterior-mean parameter values, intended to assess whether the BNN's posterior-mean predictions are physically realisable. Their scope is limited to the selected cases and does not constitute a systematic validation of surrogate generalisation.

第 2.5 节报告的直接 HF 抽查不属于后验标定流水线。它们是在后验均值参数处对耦合求解器的独立重新评估，旨在评估 BNN 的后验均值预测是否在物理上可实现。其范围限于所选案例，不构成代理泛化能力的系统性验证。

# 5. 结论 / Conclusion

This work presented a Bayesian neural network probabilistic surrogate framework for coupled multi-physics uncertainty-to-risk analysis in a heat-pipe-cooled reactor. The principal findings are:

本文提出了贝叶斯神经网络概率代理框架，用于热管冷却反应堆耦合多物理场不确定性–风险分析。主要发现如下：

(1) The BNN maintained stress R² = 0.910 and PICP₉₀ = 0.998 in-distribution, with a natural decomposition of the predictive variance into epistemic and aleatoric components that is exploited by the downstream risk pipeline.

（1）BNN 在分布内保持应力 R² = 0.910 和 PICP₉₀ = 0.998，预测方差可自然分解为认识论与偶然两个分量，供下游风险流水线直接使用。

(2) Multi-physics coupling compresses material-parameter-driven stress uncertainty by ~28% (decoupled σ ≈ 49.2 MPa → coupled σ ≈ 35.5 MPa), consistent with the expected neutronics-thermal negative feedback that is absent in decoupled analysis.

（2）多物理耦合将材料参数驱动的应力不确定性压缩约 28% （解耦 σ ≈ 49.2 MPa → 耦合 σ ≈ 35.5 MPa），与解耦分析中不存在的中子–热负反馈机制一致。

(3) Stress and keff are governed by distinct sensitivity pathways at the dominant-factor level: Eintercept (S₁ ≈ 0.56, ~59% of first-order stress variance) dominates stress, αbase (S₁ ≈ 0.74) dominates keff, with limited cross-interaction. This motivates treating stress-margin and reactivity-control material characterisation as largely independent priorities.

（3）在主控因子层面，应力和 keff 受不同敏感性路径支配：Eintercept （S₁ ≈ 0.56，约占应力一阶方差的 59%）主导应力，αbase（S₁ ≈ 0.74）主导 keff，交叉作用有限。这支持将应力裕量和反应性控制的材料表征视为基本独立的优先事项。

(4) Posterior calibration shifts the inferred parameter distribution toward observation-compatible regions (90%-CI coverage 0.889 across 18 benchmark cases, P(σ > 131 MPa | posterior) = 0.63–1.00 across the 10 near/high-stress cases for which the feasible-region table is available). Direct high-fidelity rerun of the posterior means is queued and will be reported in the supplementary revision (Section 2.5); posterior-mean vs. true stress MAE over the reference benchmark set is 6.6 MPa (bias +2.5 MPa, stress range 133–194 MPa).

（4）后验标定将推断参数分布向与观测相容的区域收敛（18 例基准案例 90%- CI 覆盖率 0.889；在 feasible_region 表可覆盖的 10 例 near/high 应力案例上，P(σ > 131 MPa | 后验) = 0.63–1.00）。后验均值处的直接高保真重跑尚在排队，将在补充修订中报告（见 2.5 节）；基准集上后验均值与真实应力的 MAE 为 6.6 MPa（偏差 +2.5 MPa，应力范围 133–194 MPa）。

The ~2.8×10⁵ single-sample MC speedup (and ~2.7×10⁸ per-sample batched speedup) over the coupled solver enables the complete pipeline — forward UQ, sensitivity analysis, and posterior calibration — to run within minutes, making risk-informed decision-making computationally feasible for reactor systems with significant material uncertainty.

~2.8×10⁵ 的单样本 MC 加速比（批处理单样本加速比 ~2.7×10⁸）相对耦合求解器，使完整流水线——前向 UQ、敏感性分析和后验标定——可在数分钟内完成，使具有显著材料不确定性的反应堆系统的风险知情决策在计算上可行。

# 参考文献 / References

[1]  McClure PR, Poston DI, Dixon DD, Cooley JW. Design of megawatt power level heat pipe reactors. Los Alamos National Laboratory Technical Report, LA-UR-15-28840; 2015.

[2]  Stauff NE, Miao Y, Cao Y, et al. High-fidelity multiphysics modeling of a heat pipe microreactor using BlueCrab. Nuclear Science and Engineering. 2024. doi:10.1080/00295639.2024.2375175.

[3]  Miao Y, Cao Y, Mo K, et al. Multiphysics simulation of KRUSTY warm critical experiments using MOOSE tools. Nuclear Science and Engineering. 2025. doi:10.1080/00295639.2025.2560170.

[4]  Aldebie F, Fernández-Cosials K, Hassan YA. Thermal-mechanical safety analysis of heat pipe micro reactor. Nuclear Engineering and Design. 2024;420:113003. doi:10.1016/j.nucengdes.2024.113003.

[5]  Jeong MJ, Im J, Lee S, Cho HK. Multiphysics analysis of heat pipe cooled microreactor core with adjusted heat sink temperature for thermal stress reduction using OpenFOAM coupled with neutronics and heat pipe code. Frontiers in Energy Research. 2023;11:1213000. doi:10.3389/fenrg.2023.1213000.

[6]  Slaughter AE, Prince ZM, German P, et al. MOOSE Stochastic Tools: a module for performing parallel, memory-efficient in situ stochastic simulations. SoftwareX. 2023;22:101345. doi:10.1016/j.softx.2023.101345.

[7]  Dhulipala SLN, German P, Che Y, et al. MOOSE ProbML: parallelized probabilistic machine learning and uncertainty quantification for computational energy applications. Journal of Computational Science. 2026;94:102776. doi:10.1016/j.jocs.2025.102776.

[8]  Lim JY, Li J, O'Grady D, Downar T, Duraisamy K. A hybrid surrogate modeling framework for the Digital Twin of a Fluoride-salt-cooled High-temperature Reactor (FHR). Nuclear Engineering and Design. 2025;433:113690. doi:10.1016/j.nucengdes.2024.113690.

[9]  Zhang T, et al. Design-oriented multiphysics analysis and surrogate modelling of a heat-pipe-cooled reactor reporting nominal peak stress ~169 MPa. Energy. 2025. [existing manuscript citation — DOI/volume to be completed by author]

[10] Blundell C, Cornebise J, Kavukcuoglu K, Wierstra D. Weight uncertainty in neural networks (Bayes by Backprop). ICML, 2015. 【待补参考：BNN 主方法引用，依据 §4.3 最终拍板方案调整】

[11] Gal Y, Ghahramani Z. Dropout as a Bayesian approximation. ICML, 2016. 【备选】

[12] Lakshminarayanan B, Pritzel A, Blundell C. Simple and scalable predictive uncertainty estimation using deep ensembles. NeurIPS, 2017. 【备选】

# 致谢 / Acknowledgements

The authors gratefully acknowledge the support provided by the National Natural Science Foundation of China [12175138]; Shanghai Rising-Star Program; and LingChuang Research Project of China National Nuclear Corporation.

作者衷心感谢国家自然科学基金（项目编号 12175138）、上海市"启明星"计划以及中国核工业集团有限公司"领创"研究项目对本研究的资助支持。

# 附录 / Appendices  (内容摘要)

A. OOD 评估（四特征尾部：应力 OOD R² ≈ 0.75–0.79 稳定；keff OOD R² 跨特征为 0.29–0.80，αbase 尾部最低；附 SST 膨胀说明） B. 确定性 HeteroMLP 基线完整测试集对比 C. 阈值扩展分析（110, 120, 131 MPa 扫描） D. 完整 Sobol 结果（8 输入 × 2 输出，含 S_T 指数与 CI） E. 全部 15 输出量精度表（FIG A5: 分输出精度图） F. MCMC 迹图（FIG A1）+ 多链 Rhat 诊断 G. 先验–后验边缘分布（FIG A2） H. BNN 校准可靠性曲线（FIG A3） I. 认识论 vs 偶然不确定性可视化（FIG A4） J. 先验–后验预测分布对比（FIG A6） K. BNN 架构示意图（FIG A7） L. 训练曲线（FIG A8）——BNN ELBO + 基线 NLL 演化 M. 速度基准详情表（TABLE M1） N. 前向 UQ predictive variant 结果对比（μ̂-only vs full posterior predictive） O. BNN 推断方案消融（MFVI vs MC-Dropout vs Ensemble，若启用）

# # DECISION LOG — 待拍板事项 (v4 BNN 迁移)  STATUS: 2026-04-15 update

# 详细分析、对比表与推荐依据见：
#   source_notes/BNN_DECISIONS.md
#
# D1. BNN 近似推断方法（§4.3 核心）：
#     ✅ 已定：MFVI / Bayes-by-Backprop
#     → bnn_model.py::BayesianLinear 使用 Gaussian mean-field +
#       reparameterization trick + 闭式 KL。
#
# D2. BNN 先验 p(w) 形式：
#     ✅ 已定：isotropic Gaussian N(0, σ²_p I)，σ_p 作 Optuna 超参
#     → 对应 BayesianLinear.prior_sigma（默认 1.0）
#
# D3. 物理约束是否保留作 BNN 的附加损失项？
#     ★ 推荐：保留。主模型选 bnn-data-mono-ineq（与 0411 主模型同构）
#     □ 备选：bnn-baseline（附录 B 作消融对照）
#     → 训练代码 run_train_0404.py 已把 w_mono·L_mono + w_ineq·L_ineq
#       加到 ELBO 之上。4 个变体已在 model_registry_0404.py 就位。
#     □ 最终定稿需等服务器训练结果出来后，对 metrics_per_output_fixed.csv
#       复核；如果 bnn-phy-mono 反超，切换到该模型。
#
# D4. 基线对比对象：
#     ★ 推荐：三列并列
#        col 1: 0411 HeteroMLP data-mono-ineq (canonical)
#        col 2: bnn-baseline
#        col 3: bnn-data-mono-ineq (本文主模型)
#     □ 风险：如 BNN R² 比 HeteroMLP 低 > 0.02，退回两列（删除 col 1）
#
# D5. 速度叙事：
#     ✅ 脚本 experiments_0404/experiments/run_speed_0404.py 已支持
#       MC-sample 推断（N_MC_EVAL 控制权重样本数）。
#     □ 待你在服务器确认 BNN_N_MC_EVAL 实际值（建议 50）并跑一次；
#       HF 基准保留 0411 的 2266 s（不必重跑）。
#     □ §3.4 改写：说明 BNN 推断时间 ≈ HeteroMLP × M，加速比相应
#       下降一个量级但仍 orders of magnitude 可接受。
#
# D6. 标题：
#     ✅ 已定改为 "Bayesian Neural Network Surrogates"（已落实至第 2/5 行）
#
# D7. Table 2 中 HeteroMLP 基线是否沿用 0411 canonical 数字？
#     ★ 推荐：沿用。数据在
#       tables/canonical_values_v3_reference.json::accuracy_fixed_eval
#       可直接读取。
#
# D8. §2.5 HF spot-check 的案例数：
#     ✅ 定论：必须重跑（BNN 后验均值与 HeteroMLP 不同）
#     ✅ 已定规模：18 例（与 §2.4 后验基准 1-对-1 配对；低/临近/高 各 6 例）
#     → 案例选择脚本骨架已写：
#        code/bnn0414/code/postproc/build_bnn_hf_rerun_manifest.py
#     → HF 重跑驱动复用：
#        code/openmc-fenics代码/run_posterior_hf_rerun.py
#
# E3. BNN_N_MC_EVAL 推断样本数：
#     ✅ 已定：M = 50（Sobol 用 M=30，MCMC 用 M=20；§4.3 已落实）
#
# E4. §2.5 HF spot-check 规模：
#     ✅ 已定：18 例（见 D8；§2.5 叙事已更新为 1-对-1 配对）
#
# E7. MCMC 多链数：
#     ✅ 已定：n_chains = 4（§2.4 与 §4.6 已加入 Gelman-Rubin R̂ < 1.1
#       收敛判据；run_posterior_0404.py 的多链代码路径已存在，
#       仅需设 N_CHAINS=4）
#
# D9. 摘要/引言是否保留 "epistemic/aleatoric 显式分解" 作为主要叙事优势？
#     ★ 推荐：保留。这是 BNN 相对 HeteroMLP 最干净的"增量"叙事。
#     ✅ 当前草稿已按此落实。
#
################################################################################
# 投稿前必须完成事项 (v4 checklist — 全部待 BNN 结果填入后复核)
# ✅ 全文定量数字已回填 (2026-04-16, 源 bnn0414/results/analysis_reports)
#   □ 待复核：HF spot-check 改为 "queued / supplementary revision" 口径
#     （见 §2.5 与 §5 Conclusion 第 (4) 条）
# □ §4.3 BNN 推断方案与先验按 D1/D2 拍板结果落实
# □ §2.1 / Table 2 按 D4 / D7 拍板结果落实基线对比
# □ §3.4 + §附录 M 按 D5 拍板结果重写速度叙事
# □ §2.5 按 D8 拍板结果更新 HF 验证案例数与统计量
# □ MCMC Rhat 定量诊断补入附录 F
# □ OOD Δ 指标（附录 A）按 BNN 新结果更新
# □ FIG 5C 联合后验密度图（BNN 后验）出图
# □ 图例 / 坐标轴 / 图标题全部遵守项目 figure-vocabulary rule
#   （禁止 iter2/level0/baseline/data-mono-ineq；禁止 CJK 字符进入图内）
# □ 摘要精简至 ≤150 词
# □ 致谢与参考文献格式核对
################################################################################